August 7, 1995                                         by N6BV

Operating Instructions for YTAD.EXE, Version 2.03

(Yagi Terrain Analysis, with Diffraction)

Version 2.03 of the YTAD program corrects several insidious

bugs that caused the computation of the gain at very low

elevation angles to be inflated artificially.

This program is a ray-tracing program to evaluate the effect

of foreground terrain on the elevation pattern of up to four

Yagis in a stack. The effect of the horizontal Fresnel ground

reflection coefficients is also modeled, meaning that the ground

conductivity and ground dielectric constant are inputs to the

program. YTAD only models horizontally polarized Yagis. The

internal antenna model is a typical 4-element Yagi. The output is

referenced however to an isotropic radiator in free space; that

is, in dBi. The free-space gain assumed for the 4-element Yagi

used inside YTAD is 8.5 dBi.

PREPARING A TERRAIN DATA FILE

The program requires an ASCII input file with the terrain data

in a particular azimuthal direction from the base of the tower.

The terrain data file is entered in feet. Several sample data

files are included with this diskette: BV-EU.PRO and BV-JA.PRO,

standing for N6BV to Europe and to Japan. These files have "PRO"

filename extensions, meaning "PROFILE." The data were taken from

a "topographic" chart (the "7.5 minute series," published by the

US Coast and Geodetic Survey.) As a reference, a FLAT.PRO file is

included as well, for flat ground.

If you examine the contents of BV-EU.PRO using an ASCII word

processor, you will find that the data is arranged in two

columns, separated by a tab; each line is terminated by a

carriage-return/linefeed. Each line represents a single point.

For BV-EU.PRO, I drew a line from the base of my tower at an

azimuth of 45 degrees. Where this line crossed each contour on

the topographic chart, I measured the distance from the tower.

0       430

200     420

500     410

700     400

800     380

900     360

1000    340

1550    360

2200    340

4800    340

5000    360

5200    380

5450    400

5700    420

7600    440

1

The first line starts at the base of the tower, at 0 feet. The

elevation height above mean sea level at my tower base is 430

feet. The next line shows that 200 feet from the tower base the

elevation is 420 feet; at 500 feet the elevation is 410 feet,

etc. I enter data at distances from the tower of more than 6000

feet, because even at that distance the effect is noticeable on

elevation pattern for low frequency Yagis, like 40-meter Yagis.

Make very sure that you terminate data input to the file with

a single blank line at the end. Two blanks lines will cause YTAD

to crash. [Note that earlier versions of YTAD required the

operator to enter a fictitious distance figure of 100,000 feet or

further at the end of a file. This is no longer necessary.]

RUNNING YTAD

Run the YTAD program by keying YTAD <Enter>. It will ask you

first for how many plots you want to overlay on the graph created

by YTAD. You can hit <Enter> for the default of a single pattern.

YTAD can overlay up to three different elevation patterns on a

single graph. I regularly compare patterns to a reference Yagi

over flat ground, usually at a high of 60 feet.

After you select the number of patterns to be overlaid

together, you are prompted for the name of a file containing

statistical elevation-angle data. This file is of the generic

form "W?-*.PRN," and comes from the disk shipped with the 17th

edition of the ARRL Antenna Book in the \ELEVAT subdirectory.

Several sample files are included with this file, but the Antenna

Book's disk contains files for all regions of the USA to Europe,

the Far East, South America, South Asia, Southern Africa, and the

South Pacific. Copy the files from the Antenna Book disk

applicable to your QTH (i.e., W3, or W4, etc.) into the

subdirectory on your hard disk containing the rest of the YTAD

files.

Next, YTAD prompts you for the name of your terrain file. YTAD

lists the *.PRO files located in the default subdirectory. Now

YTAD will prompt you to enter the frequency, in MHz, followed by

a prompt for the number of antennas in a stack. Enter 1 for a

single antenna, or up to 4 for a stack of 4 four-element Yagis.

YTAD will next ask for the height of each antenna in a stack. As

I said before, the antenna height is in feet only.

Once you have entered the antenna height on the tower, YTAD

will ask you for the ground coefficients -- if you merely hit

<Enter> twice you will enter the dielectric constant and ground

conductivity (in mS/m) for New England (i.e., very rocky soil).

You will find that for a horizontally polarized Yagi, as is

modeled in YTAD, the ground coefficients have a minor effect on

the elevation pattern at low angles, gradually becoming more

significant at higher elevation angles. YTAD does not do vertical

polarization.

2

YTAD will then compute the elevation pattern by shooting rays

from +35 degrees to -35 degrees at the terrain model in 0.25

degree increments, and then vectorially combining all the outputs

due to reflection and diffraction in the far field. The

computation will be lengthy if a stack of four Yagis is modeled

over complex terrain. If your computer doesn't have a numeric

coprocessor, the computations will be excruciatingly long...

YTAD OUTPUT

The on-screen graph will show the elevation response

(calibrated in dB down from the peak dBi level) from 1 to 30

degrees, in 1 degree increments. YTAD will also write data to a

disk file called OUT.PRN. This file contains some interesting

information about the number of reflections and diffractions that

contribute to each ray-tracing angle. You will find that complex

terrains illuminated by stacks of Yagis will produce an amazing

number of diffractions and reflections at each takeoff angle!

In addition, OUT.PRN contains the elevation data for each

angle used to create the on-screen graphs in YTAD. This data is

delimited with commas. Character labels are delimited with double

quotation marks so that data can be imported easily into a

spreadsheet for other types of manipulation, if you like.

If you want to print the on-screen graph, you will need to

load GRAPHICS.COM before you run YTAD. I use a Hewlett Packard

LaserJet IIP printer, and load GRAPHICS LASERJETII in my

AUTOEXEC.BAT file when the computer boots up. Check your DOS

manual or HELP GRAPHICS for the setup parameters for your

printer.

CAVEATS

This program is experimental. For validation of its results,

there is precious little experimental data available showing

careful measurements of HF elevation angles versus terrain

contours. The terrain data itself from topo maps is very

"sparse," in that irregularities that can be seen with the

naked eye from the base of the tower are often not shown on the

maps.

The terrain model used by YTAD assumes that the terrain is

represented by flat "plates" connecting the elevation points in

the *.PRO file with straight lines. The model is two dimensional,

meaning that range and elevation are the only data for a

particular azimuth. Obviously, the world is three dimensional. To

get a true picture of the full effects of terrain, a terrain model

which shows azimuth along with range and elevation point by point

is necessary.

The computational requirements for such a 3-D model, even

if the detailed terrain data were readily available, would be

pretty horrendous. After you have struggled to create a terrain

file by hand for a single azimuth, you will begin to appreciate

3

the data-entry problems involved in trying to come up with a real

3-D terrain map!

You can get a feel for how your terrain affects signals

launched in various azimuthal directions by creating separate

data files for these directions. For example, from my QTH in New

Hampshire, my coverage to central Europe goes from about 30

degrees to 60 degrees. I have thus evaluated azimuth shots for

30, 45 and 60 degrees.

Some of you may have access to digitized terrain databases. I

have examined the data output from one such database, and I find

that the data is relatively sparse, with rather large range steps,

and elevation data which doesn't match very well a file created

manually from a paper topo map. I suppose that the digitized data

is good enough for many military and commercial purposes, but it

doesn't seem adequate for elevation predictions using YTAD.

What would I estimate as the "accuracy" of YTAD elevation

predictions? I would say that I would trust the results within

plus/minus 3 dB. In other words, take YTAD results with a grain

of salt. Don't obsess with changing the height of your antenna by

fractions of a foot to see what happens!! However, the results of

evaluating stacks versus single antennas is VERY interesting, and

"holes in coverage" at certain useful launch angles can often be

compensated when stacks are used.

I would greatly appreciate feedback from you about this

program. YTAD is not very "friendly," because it is experimental,

and it will "crash" if your input file contains extraneous

information. Follow strictly the example of the sample files I

include. If you have access to validation data, I'd surely like

to hear about it!

In the July 1995 issue of QEX magazine, I wrote a long article

describing in detail how YTAD works. I suggest you get a copy of

that issue and read through it. Better yet -- subscribe to QEX!

There are a lot of great articles there.

GL and 73,

R. Dean Straw, N6BV

Senior Assistant Technical Editor

e-mail: rdstraw@arrl.org or CompuServe: 73063,773

regular mail: ARRL

225 Main Street

Newington, CT 06111

U.S.A.

4

###################################################################################################