CAT980                                                  Ver 2.0  Page 1

C A T 9 8 0 COMPUTER CONTROL

(Written in Turbo Pascal Ver 6)

For The YAESU(tm) FT-980

Ver 2.03

September 2, 1992

Art Harding  K5YEF

P.O. Box 1719,  Plano, Texas 75074 USA

CompuServe ID 72770,2653

Copyright 1992 - ShareWare

PUBLIC DOMAIN VERSION - MAY BE FREELY DISTRIBUTED

REASONABLE FEE FOR DISTRIBUTION IS ALLOWED

(No warranty provided or implied nor will author be liable to anyone for

any damages to data or property which may be caused directly or

indirectly by use of CAT980 programs.)

NOTE: THIS VERSION HAS ALL THE VER 1.8 AND VER 1.9 FEATURES FULLY

ENABLED PLUS MANY MORE THEN VER 1.9.  YOU ARE ENCOURAGED TO USE THE

PROGRAM FREE TO CONTROL YOUR FT-980 TRANSCEIVER.

SPECIAL VER 2.x FEATURES SUCH AS PLAIN TEXT COMMANDS AND AN AGC-SCAN

CAPABILITY HAVE BEEN RESERVED FOR REGISTERED USERS WHO HAVE HELPED

SUPPORT THE DEVELOPMENT OF THIS SOFTWARE.  THE CODE IS EMBEDDED IN THIS

PROGRAM - YOU NEED BUT THE 'KEY' TO UNLOCK IT.  YOU CAN DO THIS BY

REGISTERING BY MAIL OR FOR QUICK RESPONSE, REGISTER VIA COMPUSERVE.

SIMPLY TYPE

GO SWREG

ON COMPUSERVE AND SEARCH ON THE KEYWORD

CAT980

YOU WILL BE INSTRUCTED WHAT TO DO NEXT.  EVEN IF YOU CHOOSE TO STAY WITH

THIS PUBLIC DOMAIN VERSION, I WELCOME YOU TO SOME REAL C A T  FUN.

PLEASE SEND YOUR NAME/ADDRESS SO YOU CAN GET ON A FREE INFORMATION LIST

FOR NEWS OF CONTINUING DEVELOPMENT!

-1-

CAT980                                                  Ver 2.0  Page 2

CHAPTER ONE - CAT FEVER

Since the initial release of CAT980 in July of 1988 (Version

1.8), I have been fortunate enough to hear from a number of you. In fact

it seems anyone who had an IBM compatible and an FT-980 just couldn't

leave this program alone; they had to give it a try. And it's gratifying

that the response has been overwhelmingly positive. We are what I

thought we might be: an enthusiastic group, hungry to try new ideas. And

many of us came by our 980's mainly because of its CAT control.

Well here we are at Version 2.0 - almost a year late from my

original plan.  But I have to remind myself (and some others!) that this

is a hobby and there are other priorities out there (ain't it a

shame?!).

But I really think you're going to like this version.  It is the

first one that will respond to the FT-980 AGC line (pin 5 on the rear

panel of the CAT 6 pin DIN connector).  Of course you have to build

another interface circuit but it's no big deal.  And if you really don't

want to have the "S" meter determine where to halt a Scan (WHAT? You

don't?!) - then the program will still behave similar to earlier

versions.

And speaking of scanning, there's a brand new scan option now:

Scan Menu!  This feature like so many each version, is a suggestion from

a user.  These are your my and ideas that are going into CAT980.

And one of those ideas is the large frequency display with each

digit 3 lines high.  There are new displays as well including the

relationships between the radio filters whose relationships now are

programmable.

But one of the big features of Version 2.0 for registered users

is it's ability to understand commands typed directly from the keyboard.

Type the following and it's self explanatory what your radio will do:

HAM VFO, RIT ON, EIGHTY METERS (yes! you can just type the band you

want!), LOG, MENU, TUNE, SPLIT OPERATION, USB, LSB, FM, GEN VFO, 3.88 M

(yes! you can type in a frequency directly and it understands the M and

K multipliers!), RIT ON, UP 2.5 KHZ, WIDTH -30% (yes! now you can

command the shape the filter instantly!), SHOW STATUS, HELP - and a

whole bunch of ETC. See Appendix H.

What else?  How about changing the relationships of the various

filters?  Say what?  OK, if you are in the CW mode  and listening to a

CW signal  - then switch to USB.  The signal disappears! and that's what

it is supposed to do.  Take a moment to look at the relationship of the

various filters up on the screen. (Hey, new display!) This is the way

YAESU(tm) built the radio.

-2-

CAT980                                                  Ver 2.0  Page 3

NORMAL  | OPERATION

�����������������Ŀ

� AM/FM  |        �

�������Ŀ|�������Ŀ

�USB    �|�    LSB�

|  ����Ŀ

���ܿ� FSK�

�CW �

�������������������

3 KHz     ^      -3 KHz

YAESU Filter Model

But are we stuck with that?  Not in Version 2.0 for registered

users!  First search out a CW station in CW mode (simply type CW

<RETURN>).  Now do <AltF3> - and stop a second to take a look at what is

going on (see figure below).  Notice the signal is still there but now

there is a different filter display on the screen.  If you have the

received CW signal tuned to the back panel TONE setting - that's shown

at the top of the display and can be set in your .cfg file - then you

can now switch USB (simply type USB <RETURN>), LSB and FSK and the

signal of interest is still audible.

B F O  | 700 Hz

�������Ŀ

�USB    �

�������Ŀ

�    LSB�

����Ŀ

� FSK�

���ܿ

�CW �

�������������������

3 KHz     ^      -3 KHz

CAT980 Filter Model

What is the value of this?  Say you are in QSO on USB and

suddenly a station begins to call you on CW.  By doing an AltF3 and

switching to CW mode, you can now answer that station without doing a

wild frequency-signal hunt.  And you can go right back to USB without

losing the original station. It's experimental and I would welcome your

experiences with it.

Another experimental configuration is available, <AltF4>.  First

do an <AltF3> back to YAESU's configuration and go into the FSK Mode

(simply type FSK <RETURN>). Now type AFSK <RETURN> and here is what you

get"

-3-

CAT980                                                  Ver 2.0  Page 4

M A R K  | 2100 Hz

�������Ŀ

�USB    �

�������Ŀ

�    LSB�

����ܿ

� FSK�

���ſ

�CW �

�������������������

3 KHz     ^      -3 KHz

CAT980 'AFSK' Filter Model

You set the Mark frequency with a statement in the .cfg file.

The default is 2100 Hz which corresponds to the PK-232. Now if you have

a digital signal, say RTTY, set up in the FSK mode, you can switch to

LSB OR USB and the mark frequency is the same 'distance' from the

operating frequency as it was in FSK. This actually reverses mark and

space from LSB to USB.

FILES WITH THIS PUBLIC DOMAIN VERSION

-------------------------------------

CAT980.EXE      Version 1.9 stand-alone, executable program.

CAT980.CFG      Configuration file in ASCII that you can modify.

CAT980.DOC      This file.

*.MNU           Menu ASCII files.

*.MEM           Memory ASCII files.

APPENDIX.DOC    Contains the following appendices:

APPENDIX A      Writing The CAT980 Config (cat980.cfg) File

APPENDIX B      Writing CAT980 Memory (*.mem) Files

APPENDIX C      Writing CAT980 Menu (*.mnu) Files

APPENDIX D      Working With FT-980 TAB Statements

APPENDIX E      Working With CAT980 Color Statements

APPENDIX F      QUICK REFERENCE FOR WRITING CAT980 FILES

APPENDIX G      Interface Schematics

APPENDIX H      The Plain Text COMMANDS

ULTRA_IO.PCX         A graphic schematic of a fancy interface unit.

ORDER.DOC            How to order source code and special versions!

You probably have a computer with a hard disk.  If you do it

would be a good idea to put all the supplied files (not necessarily the

.doc files) into a a sub directory called \CAT980. Running from a floppy

is FB too.

-4-

CAT980                                                  Ver 2.0  Page 5

BUT FIRST THE HARDWARE

======================

You can use a Yaesu(tm) commercial interface, such as the

FIF-232C, or you can build the circuit described in this package

(Appendix G) for about the price of a good steak dinner - and not much

longer in time to build as to cook 'n consume.

If you do have any interface problems after going through your

attempts, then send me a SASE and explain the problem.  Don't worry

about not being a registered user; I wouldn't register either until I

had it running and planned to use it!

Besides, hamming is more important than money (not an XYL/XOM

quote).

*YOU MUST CONNECT A H\W INTERFACE BETWEEN YOUR IBM/COMPATIBLE*

*AND THE FT-980!*

If you don't have a Yaesu interface or are a builder at heart,

you should like the simple circuit in Appendix G.  It has excellent

isolation and the parts can be acquired almost anywhere including Radio

Shack(tm).  Four op amps in one chip do the job with two op amps left

over.  (You may even want to develop your own circuit.)  For a power

supply, two 9v. batteries work nicely.  Battery drain is low enough that

it may be left connected for days without going down - but it would be a

good idea to include an on/off switch or power from a permanent supply.

The author built this circuit on a Radio Shack breadboard.

*SEE 73 MAGAZINE, AUGUST 1991 ISSUE, "A UNIVERSAL CAT INTERFACE"*

The circuit is written up in more detail in 73 Magazine in an

article by this writer called UNIVERSAL CAT INTERFACE.  A single supply

chip circuit is also presented.  Now I'm not saying you need this to get

this circuit going, on the contrary I believe you can build it without

the 73 article.  Nevertheless you can get reprints of the article from

the publisher for $3.00 1st copy, $1.50 each additional copy (get your

grandmother one). Ask for the article by name and date.  Send to:

73 Magazine

WGE Center

Forest Road

Hancock, NH 0344

Look for a quite usable version of the article on CompuServe

too!

-5-

CAT980                                                  Ver 2.0  Page 6

GETTING STARTED

===============

If there is going to be any trouble at all it will probably be

in the H/W.  If you are using the commercial interface then you're way

ahead of the game; even the cables are ready to go.  It should be a

piece of cake. People using the FIF-232 have confirmed this.

If you're a mama-please-I'll-do-it-myself, like the author, then

be careful about the pin-outs for both the computer and the radio (see

Appendix G).  Once you get it running it will be a snap from then on.

But please don't get discouraged if you do have trouble.  THIS DEAL WILL

WORK and give you hours of fun. Have the persistence to stick with it;

dealing with COM ports is always tricky.  But many have been through it

before you, so hang in there!

OK, we'll assume at this point you are interfaced and your cable

is plugged into the back of the computer in the com1 connector.

* LET'S STAY IN DOS FOR THE INITIAL SET UP *

* WE CAN WORK IT OUT IN WINDOWS ETC. AFTER IT'S COOKING *

* BUT STOP!  Do You Really Want To Use COM 1? *

The default port for CAT980 is COM 1.  But many people have a

mouse, packet modems, etc., hanging on this port.  CAT980 let's you use

any of 4 ports - but you must tell the program which one you want to

use.  You can do this by adding the statement COMM=2 (or 3 or 4) to the

supplied ASCII file CAT980.CFG for the port you want to use.  See

Appendix A for a discussion on this - but not right now -  for now you

can ignore the port assignment because there is an easier way to get

started.

In a moment you will be asked to bring up CAT980. Upon entry to

CAT980 you may hit a Control P <^p> and bring  up a port menu which will

allow you to choose any port at that time but it must be the first thing

that you do. If you are indeed connected to com1 then that's the default

and you don't even need the <^p> deal.  If you are connected to another

com port, then choose that one from the menu.

You should add the port statement to the .cfg file later when

you're sure you're happy with a particular configuration. Once you do

add it, a <^p> won't let you change ports any longer. So wait to add

this feature.

-6-

CAT980                                                  Ver 2.0  Page 7

CHAPTER TWO - TAKING A TOUR

Initial FT-980 Settings

=======================

Let's take a 'tour' through CAT980. This documentation may tell

you to type a command (like USB) from time to time.  Please remember

that if you are not a registered user and do not have the 'key' these

instructions will not work for you.  But no matter; just follow on

there's plenty of fun here without being registered (this is a hobby

after all!).

And before we start the program let's set up the radio.  This

will not be necessary in the future, this is only for this first time so

the demonstration we are about to go through will go smoothly.  So, from

the front panel choose/push/switch/dial the following:

1. HAM

2. VFO

3. USB

4. 14.250.00 (20 meters)

5. Width/Shift Dials Centered

6. Clarifier (RIT) off.

7. Tabs off.

8. LDB off.

9. MEMORY channel switch to channel 1.

FROM THE BEGINNING

==================

OK, if the hardware is in place then let's bring up the program

from the directory where it resides. At the DOS prompt type:

cat980   <CR>

EXIT?

-----

Ever get a new piece of software going and then realize you have

no idea how to get out of it?  Well, before you get into this here's how

you get out: Hit <Alt X> and follow the exit menu instructions. (More

about these options later in this document.)

CAT980 UP AND RUNNING

=====================

The first thing you will see is the header with the copyright

shareware info.  Exciting, isn't it?  No?  Then either do a <^p> to pick

a com port other than ONE, or press any key to accept COM1 and move on.

-7-

CAT980                                                  Ver 2.0  Page 8

The next thing you'll see happen will be a message stating that

the .cfg file is being read(or that it's not present, it really doesn't

have to be but it should be there for this tour).   That's the file

mentioned above with the port info and some other customized

installation info.  There's more to that file - like a zulu time shift

statement and a way to show your time zone -  but we'll get to that

later.  You'll be able to get your hands on all these thing but for now

let's continue.

The only thing that will abort the program at this point is a

serious bum statement in the .cfg file - like: COMM=5. Most errors in

the .cfg file will simply be brought to your attention on the screen but

the program will continue after your acknowledgement.

PROOF OF THE PUDDING

====================

After the screen tells you it is done reading the .cfg file it

will ask you to hit any key.  Hit a carriage return <RETURN>.

*IF YOU DON'T GET A H/W ERROR MESSAGE NOW YOUR INTERFACE IS A.O.K!*

One of two things is about to happen.  One - the program will

continue and you can congratulate yourself on a proper hardware

installation.  Two - a hardware port error message will appear and the

program will automatically abort.  Don't be discouraged if this happens;

the original system development had both software and hardware unknowns

to contend with.  Go back over the lash up.  If you have an RS 232

tester you should be able to see CAT980 trying to communicate with the

radio.  Pin connections, mixed or wrong, are usually the culprits.

(NOTE: A Radio Shack(tm) 'RS-232 TROUBLESHOOTER', 276-1401, is a handy

tool you can use to actually see the data being transferred. Handy, but

not necessary.)

Now there is a software adjustment in the .cfg file that relates

to port behavior: PortTime=x and is covered in Appendix A.  If you have

a very very fast computer this parameter can help.  BUT - it's not

recommended that you turn to this avenue before you are absolutely sure

that your hardware is working and hooked up properly. (Did I just hang

out a WET PAINT sign?)

If you really get into what seems an unsolvable interface

problem then drop me a line on CompuServe(ID 72770,2653) or in the

mail(sase) - and please document your problem thoroughly.

-8-

CAT980                                                  Ver 2.0  Page 9

ON WITH IT

==========

Once you get past the interface installation the program can go

into its next phase.  For first-time demonstration purposes it will load

a .mem file into the FT-980 and you can see this happening on the

screen.  It will load a .mem only if the .cfg file tells it to do so

(the one on the disk tells it to load examp20.mem) - again, more about

that later.

What you will see is a presentation of the FT-980 16 memories

(yes, 16, not 12 as shown on the front panel).  As you watch you will

see Channels 1, 8, 12, and 14 thru 16 being loaded by examp20.mem file

on your disk.  (You will be able to write numerous such files which will

load all or any combination of the 16 memories.)

*DON'T WORRY. YOUR ORIGINAL STORAGE IN ALL CHANNELS*

*WILL BE RESTORED WHEN YOU EXIT CAT980!*

Next you will see a message indicating that storage frequency

slots are being loaded.  This particular file will load 235 special

memories inside CAT980.

You will learn how to write such files later.  For now wait

until you are asked to hit any key and please do so. Ahh - another

decision to be made:

Install FT980 Memory Channel 1 into VFO? [Y/n] [ESC=Abort]

What this means is, do you want to leave the Memory Channel as a

memory channel and install it as such (answer 'n') - or do you want to

translate the frequency, mode and vfo stored in channel one directly

into the radio - something the FT-980 wouldn't let you do.  So hit Y

(you can type a Y or just do a <RETURN>.

Note that your radio is no longer on Channel 1, it is on the HAM

VFO, USB and the frequency that is stored in the FT-980 memory channel

1.

Now let's go back to that 235 memory load that CAT980 did. Just

for grins let's take a look at what is stored in these 300 memory slots.

Do that by hitting a Control F <^f> or typing the command: SHOW SLOTS

<RETURN>. You have to use the <PageDown> key to see 'em all. Hit a

<RETURN> or <ESC> when you're through looking at these slots.

-9-

CAT980                                                 Ver 2.0  Page 10

See what happened? What the file examp20.mem has done is to load

a slot every 100 KHz from 3.5 MHz thru 29.9 MHz. (Don't transmit!)

THE WORKING SCREEN DISPLAYS

===========================

NORMAL  | OPERATION

�����������������Ŀ                 �COM 1�   �����Ŀ

� AM/FM  |        �                 � PC  ��﮳FT980�

�������ܿ|�������Ŀ                 �������AGC�������

�USB    �|�    LSB�

|  ����Ŀ         ������������������������ķ

���Ŀ� FSK�         �  ACTIVE FILES LOADED   �

�CW �               �  -------------------   �

�MeNU:   DEMO20.MNU      �

�������������������        �LOG:      (none)        �

3 KHz     ^      -3 KHz     �MEMory: EXAMP20.MEM     �

YAESU Filter Model        ������������������������Ľ

20 M

14.35     14.30     14.25     14.20     14.15     14.10     14.05     14.0

�         �         �         �         �         �         �         �

���������������������������������������������������������������������Ͼ

^

��  � �   ��� ��� ���   ��� ��� ���

�  ���   ��� ��� � �   � � � � � �                    �Thr����23�APR�92�

���   � � ��� ��� ��� � ��� ��� ���     ���������Ŀ    �17:49:06 �CST   �

�Vfo > HAM�    �23:49:06 �Zulu  �

�� Mhz        �� Khz   OP FREQ  �Mode> USB�    �Thr����23�APR�92�

COMMAND >  _                           �����������

TYPICAL CAT980 VER 2.0 SCREEN DISPLAY

Before we go any further let's take a moment to look at the

screen displays a little closer.   Starting at the bottom is the

"COMMAND >" line where you will be able to type in plain text commands

and most error message will appear on this line.  Just above this line

on the far left is the OP FREQ, the same digital display as on the

radio.  Both the megahertz and kilohertz digits are marked for an easy

read.

�����������������Ŀ          ����������������Ŀ

�Vfo > HAM MEM ^ 1�          �Vfo > HAM MEM  1�

�Mode> USB        �          �Mode> USB       �

�������������������          ������������������

Memory Unlocked              Memory Locked

STATUS BOXES - MEMORY MODE

Moving immediately to the right the next box is the status

display box which displays which vfo is in control: HAM or GEN. The

display will show MEM, if in memory mode along with the selected

-10-

CAT980                                                 Ver 2.0  Page 11

channel.  A little triangle just before the channel number indicates

that the memory is unlocked (as SHIFT/LDB button on the radio does) or,

if absent, that the radio is locked to the memory channel, the 'normal'

memory state. You can toggle the lock with your <Delete> key; when it is

unlocked you can change the frequency but of course not the mode. It

will have the same effect as pushing the LDB button on your radio when

it is not captured.  If you're in VFO and not MEM, then the <Delete>

button doesn't do anything but produce an error message. (Ah, there's so

much to learn and so little time'  Shakespere - I forgot what his call

was. G0 something?)

�����������������Ŀ

� Vfo > HAM       �

� Mode> FSK 170 Hz�

�������������������

STATUS BOX - FSK MODE, 170 Hz TONE SHIFT

OK, in the same box on the bottom line is the Mode - LSB, USB,

CWW, CWN, AMW, AMN, FSK or FM.  If in FSK the shift, 170, 425 or 850 hz

is also shown.

������������������Ŀ

�RIT Rx> +  100 Hz �

�Vfo > GEN         �

�Mode> USB         �

��������������������

STATUS BOX - RIT ON, 100 Hz HIGH

Also in the same box is the 'clarifier' (that's Yaesu's name) or

more popularly called the RIT indicator.  This top line only comes alive

if the RIT is activated.  It indicates RX, TX or RX/TX configurations

and displays + or - shift in hz from the OP FREQ. If there is no shift,

no number is shown.  If RIT is OFF no display clarifier info is shown.

�Thr����20�DEC�90�

�23:32:52 �CST   �

� 5:32:52 �Zulu  �

�Fri����21�DEC�90�

CAT980 ON-SCREEN LOCAL/ZULU CLOCK

The next display box is the Time Box.  It shows the day and date

for local time plus a local time zone descriptor you can supply in the

config file (see Appendix A).  Below the local time is UTC (zulu) which

is displayed as a shift from local time.  This time differential is also

set up in the config file by you to customize the time at your QTH.

20 M

14.35     14.30     14.25     14.20     14.15     14.10     14.05     14.0

�         �         �         �         �         �         �         �

���������������������������������������������������������������������Ͼ

^

CAT980 20 METER ANALOG DIAL

-11-

CAT980                                                 Ver 2.0  Page 12

Above the status boxes you will see an analog presentation of

the FT-980 dial.  Each ham band has its own dial display and there's a

special full-spectrum display one for the GENeral VFO.  The dial

initially shown will be whatever the radio was tuned to before it was

captured.  If you were on 20 meters than that's what will be displayed.

NORMAL  | OPERATION

�����������������Ŀ

� AM/FM  |        �

�������Ŀ|�������Ŀ

�USB    �|�    LSB�

|  ����Ŀ

���ܿ� FSK�

�CW �

�������������������

3 KHz     ^      -3 KHz

YAESU Filter Model

Now look center screen, directly above the analog dial for the

'NORMAL OPERATION' - 'YAESU Filter Model' mentioned at the beginning of

this document.  Toggle <AltF3> and <AltF4> to view the various

combinations.  These features are for the advanced CAT operator (and

registered user) which hopefully you will eventually become.  For now

toggle back to the YAESU model.

������������������������ķ

�  ACTIVE FILES LOADED   �

�  -------------------   �

�MeNU:   DEMO20.MNU      �

�LOG:      (none)        �

�MEMory: EXAMP20.MEM     �

������������������������Ľ

Next take a look over to the right at the 'ACTIVE FILES LOADED'

display.  The supplied config file (CAT980.CFG) told the program to load

the MEMory file EXAMP20.MEM and you saw the result of that action as the

program was first loaded.  It also told the program to load a menu,

DEMO20.MNU, too.  That's an exciting feature we will get to in short

order.  For now let's have a sneak preview by hitting the space bar

<space>. No, no, don't mess with it just yet (oh yea? try and stop me!)

- please just hit the space bar again to stand-down the menu.

The .cfg file did not load a log file simply to demonstrate what

the display looks like when such is the case.  We'll load one later in

this exercise.

-12-

CAT980                                                 Ver 2.0  Page 13

This file status box can be turned off and on from the .cfg and

other files as well.  To toggle the display from the keyboard use

<AltF6> - go ahead and try it.

The last display box just above the file display shows an

indication of the PC/FT980 interface condition. An X means no

communication between these two - which is normal when the program is

first called.  It will be replaced by an upside down U, �, or horse shoe

after CAT980 captures the radio.  This box also displays the port in use

(COM 1 thru 4) after the radio is captured.

Just above the box will appear an indication if you raise the

RTS or DTR lines.  Nothing will show if the lines are in their default

low state. (See TAB and ShftTAB).  You may choose to build a keyline

circuit such that <TAB> will turn on the transmitter.  Drawings are

provided for this in Appendix G or in the included ULTRA_IO.PCX file.

Finally, below the 'horse shoe' is the space which will indicate

wether or not the FT-980 AGC line is being monitored.  If not, nothing

will be displayed, if so then the letters 'AGC' will show this feature

is armed and ready to scan.  This too can be turned off and on by

various CAT980 files. <AltF5> will toggle the feature from the keyboard.

Of course special hardware will be needed to realize the true

benefit of the AGC connection.  But not that special; take a look at the

simple circuit in Appendix G or use your paint program to print out

ULTRA_IO.PCX.

TRY IT OUT

==========

OK, for the next stop in our mini tour let's try some or all of

the frequency commands below:

Hit Key         Key Notation    CAT980/FT-980 Action

-------------   ------------    --------------------

Right Arrow    <RtArw>        Down 10 hz

Left Arrow     <LtArw>        Up 10 hz

Up Arrow       <UpArw>        Up 100 hz

Dn Arrow       <DnArw>        Dn 100 hz

PageUp         <PgUp>         Up 1 Khz

PageDown       <PgDn>         Dn 1 Khz

Ctrl Lft Arrow <^LtArw>       Up 5 KHz

Ctrl Rgt Arrow <^RtArw>       Dn 5 KHz

Plus           <+>            Up x KHz (you set up freq. diff)

Minus          <->            Dn x KHz (you set up freq. diff)

Home           <Home>         Top of present band.

End            <END>          Bottom of present band.

Ctrl PageUp    <^PgUp>        Change Band Up

Ctrl PageDn    <^PgDn>        Change Band Down

-13-

CAT980                                                 Ver 2.0  Page 14

Home/End Moves frequency from one end of the band to the other.

On the GEN VFO this is from 29.999.990 Mhz to 150 Khz!  On the ham bands

it goes from one legal end to the other. It will also honor limits set

by the FT-980 Tab features. Chances are you haven't been using the

FT-980 TAB capability up to now.  There's a whole appendix devoted to

utilizing this most helpful feature of your radio. (Appendix D).

If you want to go to the center of a band try a Control J <^j>.

And here's a taste of things to come for registered users.  Instead of

hitting <HOME> type the word TOP and a carriage return <RETURN> or <CR>.

Then try the word BOTTOM and then try the word MID.  Appendix H has

quite a long command list which CAT980 understands. But like we're

inclined to say, 'more on this later'.

*IT DIDN'T WORK?*

*YOU CAN'T PICK UP SIX METERS ON THE FT-980!*

Remember, the radio itself has inherent limitations.  As eluded

to above, if you captured the FT-980 while it was in memory mode than

you can't change the frequency from the keyboard any more than you can

from the front panel if you choose to leave it in memory mode.  You have

to unlock the memory (SHIFT/LDB button on the FT-980) - you do this with

the <Delete> key.  If this is the problem then try it and then run thru

the above keys.

*STAY AWAY FROM THE FT-980 CONTROLS?*

*(JUST SOME OF 'EM, THAT IS!)*

By the way, if habit has you reaching over and pushing buttons

on the FT980 after it's captured, nothing may seem to happen.  BUT -

when you exit CAT980 all those buttons you pushed will then attempt to

take effect.  The knobs you can twist are those that are not allowed to

be controlled by the radio's CAT PROM.  You'll learn what you can and

cannot do quickly. In general:

FT-980 FRONT PANEL

---------------------------------------------------------------------

|YOU HAVE CONTROL OF ALL KNOBS AND  |      CAT980 HAS CONTROL OF ALL |

|BUTTONS('LOCK' excepted) TO THE    |      KNOBS ('FWD SET'/'ATT'    |

|LEFT OF THE MAIN TUNING DIAL       |      excepted) AND BUTTONS TO  |

|                                   O      THE RIGHT OF THE MAIN     |

|                                   |      TUNING DIAL.              |

----------------------------------------------------------------------

*CAT980 HAS CONTROL OF THE MAIN TUNING DIAL*

-14-

CAT980                                                 Ver 2.0  Page 15

Let's bring up the HELP displays; this is what you'll be running

with for a while but you'll need them less and less as you operate.  Hit

<F1>.  This is a four page display which shows most of the features and

many of the commands.  The frequency commands we played with above are

shown here plus a whole lot more. Do a <PgDn> to read more of the menu.

<ESC> will take you back to the command line when you're done exploring.

We're going to stop our tour bus at this point.  We really

haven't begun to explore it all but you've had enough hands-on to create

you own tour at any time you get the itch.

Certain features such as scanning (Chapter Four) have a whole

chapter devoted to the subject.  And many options have given birth to

the Appendix set.

The next chapter is a list and explanation of all the CAT980

commands.  It starts off with the EXIT COMMAND in case you'd like to QRT

about now.

-15-

CAT980                                                 Ver 2.0  Page 16

CHAPTER THREE - THE CAT980 COMMAND SET

THE CAT980 COMMAND SET

======================

The following is a list of the CAT980 major commands and an

explanation of their effect.  The presentation format for each command

shown is:

������������������������������������������������������������������Ŀ

�<KEY TO HIT>    'PLAIN TEXT CMD'          "Command Name"          �

�                -[Registered Users]-                              �

�This is an      This is what you can       The short name for the �

�actual key      type directly from         command like:          �

�like <F1> to    the keyboard to get the    "CAT980 HELP FILE".    �

�hit.            same result. Like 'HELP'.                         �

��������������������������������������������������������������������

On plain text commands the upper case letters shown must be typed, the

lower case are optional.  In the F2 frequency command, for example, the

word FREQ has the same effect as the word FREQUENCY.

************************************************************************

<AltX>          'QUIT'            "Exit Command" Menu

When you do an Alternate X you will be given the option of a

number of exit procedures.  The <CR> will remove CAT980 totally and

releases the FT-980 from capture.  A <Z> will do the same except for one

important difference: the FT-980 remains captured and 'frozen' on all

settings even though CAT980 is out of the picture. (Notice the EXT light

on the radio remains on.)  An <X> really isn't an exit but a side trip

explained later.  And our friend OOOOPS is realized by <ESC>.

����������������������� Exit Options �������������������ͻ

�                                                        �

�<CR> Release CAT Control of Radio - Return to DOS       �

�                                                        �

�<X> Exit to DOS Shell - CAT980 remains intact in memory �

�                                                        �

�<Z> Exit (Leave Radio Captured) - Return to DOS         �

�    [CAT980 will be removed from memory]                �

�                                                        �

�<ESC>  Abort Exit Procedure                             �

��������������������������������������������������������ͼ

-16-

CAT980                                                 Ver 2.0  Page 17

Related Plain Text Commands:

'BYE'                           This will bypass the menu, release the

FT-980 from capture and leave the

program.

'EXIT'                          This will leave the program but the

FT-980 will remain captured.

************************************************************************

<ESC>           (none)          "Abort and Ooooops"

The <ESC> key can be your friend.  It will often let you back

out and/or abort a procedure that has been started.  When in doubt use

it until it does nothing else but produce an error msg. An <AltX> often

performs the same function as <ESC>.

************************************************************************

<F1>            'HELP'          "CAT980 Help File"  Display

You will reach a proficiency level where you won't need this

display, but it's always there if you do. Four pages make up the help

screens and <PgUp> <PgDn> moves you thru the displays. <F1> or <ESC>

removes the help screen.

************************************************************************

<F2>           'FREQuency'      "Key In A Frequency"  Display

This allows you to key in a frequency with numbers from the

keyboard.  If you hit an <ESC> at any time the procedure will abort and

the original frequency that was initially displayed will return.  Even

if you have entered some digits.

����������Mhz���������Khz������������ͻ

�  ��  � � ��� ��� ��� ��� ��� ���    �

�   �  ��� ��� ��� � � � � � � � �    �

�  ���   � ��� ��� ��� ��� ��� ���    �

�                                     �

�������������������������������������ͼ

But if you hit a <CR> at anytime the display is showing what you

want - regardless of where the cursor is - that's the frequency you will

get.  You can use the right/left, space, and arrow keys to move about

and even the <Home><End> keys will take you to each limit. (<Space> acts

like a right arrow.)  But keep in mind that if you try something the

radio just can't do, you'll get an error, - like trying to key in 8 Mhz

from one of the ham bands.  The bands and dial display will switch to a

new band display, however, if the frequency you key in is valid and

requires a new view.

-17-

CAT980                                                 Ver 2.0  Page 18

*ERROR - Beyond FT-980 Capability*

*THE RADIO CAN'T GO TO 6.99 MHZ IF IT'S ON 40 M. HAM VFO!*

Related Plain Text Commands:

Simply type in the desired frequency on the command line and hit a

<RETURN>. The multiplier M for MHz and K for KHz is recognized by the

program. For instance the following three commands all give the same

result for 20 meters:

14150000

14150 K

14.15 M

************************************************************************

<F3>            'MODE'          "Mode Options" Menu

LSB USB CWW CWN AMW AMN FSK FM

This is rather straight forward.  You use the left/right arrow

and the <Home><End> keys to move from mode to mode. Note that the

command is instant and follows the arrow keys so that you can "hear"

what you're doing.  (Remember, you can't change the mode in memory - or

can you?  See <ShftF9>!) When you have the mode you want, hit a <CR>.

Note also that if you choose FSK another display appears and asks you

for the shift.  Now you can go from 170, 425 to 850 hz without crawling

behind the rig looking for that slide switch!!

Like many of the commands you can 'back out' of the Mode Options

Menu by simply hitting the command key that produced the display,, in

this case <F3>, and it will return you to the command line.  (<ESC> does

the same thing.) This comes in handy when you hit the wrong key.

Related Plain Text Commands:

'LSB' 'USB' 'CW' 'CWW' 'CWN' 'AM' 'AMW' 'AMN' 'FSK' 'FM'

The results of these commands are immediate.

************************************************************************

<F4>            'HAM'                   "HAM/GEN Toggle"

'GENeral'

This is another straight forward command.  If you're in the VFO

mode hit <F4> to change from GEN to HAM and vice versa.  If the radio is

in the memory mode this key will, of course, give you an error. (Select

VFO and try again. See <F5> next.)

************************************************************************

-18-

CAT980                                                 Ver 2.0  Page 19

<F5>            'CONFIGuration' "VFO MR RXM RXV Options" Menu

<F5> brings up a menu which allows you to go thru the four major

VFO|MEMory options offered by the FT-980.  Use the left/right arrow keys

again to move about (and <Home><End>) - to move thru the various

configurations - again they happen real-time as you move the cursor.

But notice how the analog display really shows you what the deal is

(unlike the radio!).

You can see the splits and what the Tx frequency will be before

you key up!  A <CR> makes your selection 'permanent'.  As with the front

panel operation you must make sensible choices for split operation.

*SPLIT OPERATION - LIKE DX ON 40 METERS*

*WANT TO HEAR IF THE FREQUENCY IS CLEAR BEFORE YOU TRANSMIT?*

** USE THE <\> AS A SUPER FAST TOGGLE **

The <\> key allows you to instantly switch between VFO and

MEMory without evoking the menu so you can instantly listen on the TX

frequency before you begin your transmission.

Related Plain Text Commands:

'VFO' 'MR' 'RXM' 'RXV'

************************************************************************

<F6>            'RIT'           "Clarifier Options" OFF RX RX/TX Menu

Use the arrow keys to move about and change the RIT through four

different operating possibilities.  After you make your choice <CR>,

most of the frequency controls move the RIT frequency.  RIT will also be

disabled automatically by some of the other commands - like a band

change from 80 to 40 meters will turn off any RIT settings and shifts.

Note that such shifts are not lost when the WWV/Time key <F10> is used.

** USE 'CONTROL \'  <^\> FOR A QUICK WAY IN AND OUT OF RIT! **

<Ctrl\> toggles the Rx RIT without calling the RIT menu. This is

a helpful tool for DX'ing and those round table QSO's where one station

always seems to be off frequency. (Murphy's Round Table Law.)

Related Plain Text Commands:

'RIT ON' 'RIT OFF'  'RIT TX'

************************************************************************

-19-

CAT980                                                 Ver 2.0  Page 20

<F7>            'MEM980'        "Radio Memories Active Display" / Menu

�������������������������������Ŀ

�   M E M O R Y  S T O R A G E  �

���������������������������������������������������ͻ

�1: 14.250.000 USB HAM   �    9:  9.755.000 AMW GEN �

�2: 28.980.000 USB HAM   �   10:  9.580.000 AMW GEN �

�3:  3.995.000 LSB HAM   �   11:  0.570.000 AMW GEN �

�4:  1.190.000 AMW GEN   �   12:  1.080.000 AMN GEN �

�5: 16.408.200 USB GEN   �   13:  7.000.000 LSB GEN �

�6:  9.590.000 AMW GEN   �   14: 29.400.000 USB HAM �

�7:  9.410.000 AMW GEN   �   15: 29.600.000 FM  HAM �

�8: 14.103.000 FSK HAM   �   16: 29.660.000 FM  HAM �

���������������������������������������������������ͼ

TYPICAL DISPLAY OF THE FT-980 INTERNAL MEMORIES

(Note there are 16 memories in the 980 - NOT 12!)

Hitting <F7> will activate the Memory display.  The first time

it is called the display is actually a menu with its own command set.

The command line relinquishes control and the applicable commands in

effect are displayed on the left side of the display and now apply only

to this display. The arrow keys and <Home><End> keys will move you about

the memory display.

If the radio is in memory mode upon entry then the channels will

become active - actually change -  as you move about.  You can hear it.

When you hit a <CR> the memory channel the cursor is on becomes active.

Note again that while the front panel only has 12 memories, the radio

actually offers 16.

When you choose a channel with <CR> one of three things can

occur.  The radio will either go directly to that memory channel in the

memory mode, put the memory information into the VFO and switch to the

VFO mode, or the command line will prompt you as to which one you wish

to do. You can set up what you want as your options to happen, with a

line in the config file (See Appendix A) or use a <^e> to change these

options from the keyboard.

When you hit <F7> again (or <ESC>) the display of memories will

remain but the menu feature is disabled and all command keys go back to

their regular state on the command line.

<ESC> or <F7> turns off the display.

Related Plain Text Commands:

'SHOw MEMories' Brings up FT-980 memory display but NOT active.

'PUT'           Installs present FT-980 memory channel into the VFO.

'GO xx'         Goes directly to FT-980 memory channel xx (1-16).

'CHANnel xx'    Same as GO xx.

************************************************************************

-20-

CAT980                                                 Ver 2.0  Page 21

<F8>            'FILter'        "IF Filter Active Display" Menu

���������������������������������������������������������������������Ŀ

� Active Display COMMANDS: Arrows {Up/Dn/Home=SHIFT}{Lf/Rt/End=WIDTH} �

�                                                                     �

�                               op freq.         Variable Filter (USB)�

�                                  |                                  �

�        .......................   |                Band-pass 2500 hz �

�       \                       /  |                Shift        0 hz �

�      .                         . |                Width    0.00 %   �

�     .                           .|                                  �

�    .                             |                                  �

�    |----|----|----|----|----|----|----|----|----|----|----|----|    �

�  3.0   <--------norm----------> 0.0                           -3.0  �

�����������������������������������������������������������������������

TYPICAL FILTER DISPLAY MENU

This feature displays an approximate but reasonably close plot

of the variable band pass of the FT-980 I.F. filters.  The command line

relinquishes control and a new command set menu takes over so you can

manipulate the filter.  This takes the guess work out of tuning by ear -

in fact it actually compliments your hearing.  Pertinent filter data is

calculated and displayed as a plot in relation to the fixed band pass.

This display is not constructed by commands to the filter, but is a

representation of the data actually derived from the FT-980 PROM status

stream updated after each command.

�������������������������������������������������������������������Ŀ

�Variable Filter (CWW)          op freq.                             �

�                                  |                                �

�Band-pass  600 hz          .......|...                             �

�Shift        0 hz         \       |   /         |                  �

�Width  100.00 %          .        |    .        |                  �

�                        .         |     .       |                  �

�                       .          |      .      |bfo (700 hz)      �

�    |----|----|----|----|----|----|----|----|----|----|----|----|  �

�   1.5                 .5  <--norm---->    .5                 -1.5 �

���������������������������������������������������������������������

TYPICAL CW FILTER DISPLAY

When displaying the CW filters a 'BFO' carrier is positioned on

the plot in relation to the band pass. This is for reference only; this

is a pseudo carrier presentation and its position on the screen depends

upon the position of your 500/600/700 Hz CW switch on the back of the

FT-980 and a line in your config file.

-21-

CAT980                                                 Ver 2.0  Page 22

���������������������������������������������������������������������Ŀ

� Display  C O M M A N D S: Arrows {Up/Dn/Home=SHIFT}{Lf/Rt/End=WIDTH}�

�Variable Filter (FSK)          op freq.                               �

�                                  �     14.100.875 mark  space       �

�Band-pass -2500 hz                �            .���������|������.    �

�Shift      300 hz                 �           \2125 hz � |2295 hz/   �

�Width� -28.35 %                   �          .         � |        .  �

�                                  �         .          � |         . �

�                                  �        .           � |          .�

�    �����������������������������ϳ����������������������������Ͼ    �

�  3.0                            0.0        �--------norm----------� �

�����������������������������������������������������������������������

TYPICAL FSK FILTER PLOT

Another special display appears when the FSK mode is chosen.

CAT980 determines which shift is being used, 170 hz, 425 hz or 850 hz

and then displays the mark and space positions in relation to the filter

pass band. Actual mark frequency is also displayed. Notice the shift

between this reading and that of the front panel or PP FREQ. screen

display. If you are in the AFSK filter configuration <AltF4> then this

display will be modified accordingly.

(NOTE: The calculations necessary to plot actual filter characteristics

and skirt behavior turned out to be a rather tricky exercise for this

ham!  For those inclined to know more about this subject and how our

radio behaves, please grab a keyboard and write me.)

Hit <F8> again (or <ESC>) and the filter display remains but the

menu feature closes and all command keys return to their regular state

at the command line.  To see all the filters in this state hit an <F3>

(provided the radio is not in memory mode) and 'walk' through the

various modes.  The filter display will change accordingly.

<F8> or <ESC> will remove the display.

Related Plain Text Commands:

'SHOw FILter'   (Brings up FT-980 memory display but NOT active).

'DISplay FILter' Same as SHOW FILTER.

'CAT980'        Shifts filter relationships based on CW tone.

'AFSK'          Shifts filter relationships based on AFSK mark tone.

'FT-980'        Reset filter relationships to Yaesu's configuration.

************************************************************************

-22-

CAT980                                                 Ver 2.0  Page 23

<F9>            'SCAN'           "SCANNER Options" Menu

�������Scan Options�������ͻ

� Scan FT-980 Memories     �

� Scan Band                �

� Scan Marked Frequencies  �

� Scan Menu Selections     �

��������������������������ͼ

There are four different scan modes - each accessible through

<F9> or individual control keys from the command line.  From <F9> use

the up/down arrow and <Home><End> keys to select the desired scan mode.

A <CR> will activate your choice. Or use:

<^n>      'SCan MEMories'         Scan FT-980 Memories

<^b>      'SCan BAnd'             Scan Band

<^v>      'SCan SLOts'            Scan Marked Frequencies

<RETURN>  'SCan MENu'             Scan Menu Selections

Earlier versions of CAT980 required operator intervention to

stop a scan. Now if you build the simple circuit in Appendix G then the

signal level of the received signal (AGC line) will automatically stop

the scan at a pre defined 'S' level setting of a pot. (This feature is

enabled only for the registered user.)

Depending upon the scan selection you have made you will be

prompted to supply other parameters.  See "Scanning Operations"

described latter in this document for more details.

(Having trouble remembering all this 'stuff'? Remember your pal,

the <F1> function key.)

************************************************************************

<F10>           'WWV'           "WWV 10 Mhz" Check & Return

'TIME'

Here's a real help for everyday operating.  Just hit <F10> and

you've got WWV on 10 Mhz.  Hit it again (or any key) and you go right

back to the operating frequency and mode prior to the WWV command.  If

10 Mhz propagation is not cooperating then hit <Shift F10> and you get

your programed choice of a time frequency station with the same instant

return feature.  (See <ShftF10>.)

Related Plain Text Commands:

'ALTernate time'                Yields a secondary time standard freq.

************************************************************************

-23-

CAT980                                                 Ver 2.0  Page 24

THE SHIFTED FUNCTION KEYS

=========================

<Shift F1>      'FILEs'         "Files" - Calling In CAT980 User Files

This command allows you to load three types of operating files:

memory, menu and log files that you have created.  Be sure and load only

files with extensions .mnu, .mem and .log - although CAT980 may permit

other extensions to possibly be called in as .mnu.  You will be warned

with a prompt and it's not recommended to name your files with other

extensions. Stick with .mem for memory files and .mnu for menu files.

To realize the full advantage of this feature read Appendix C.

For now do a <ShftF1> and load the supplied 10 meter beacon frequency

memory file.  Supply the prompt with 'TENRPT20.MEM' without the '' marks

and hit a <CR>.  There are some more .mem files on your disk you will

want to try.

������������� Frequency Bin Storage - 20  Slots Loaded �������������Ŀ

�  1 � 29.500.000   2 � 29.510.000   3 � 29.520.000   4 � 29.530.000 �

�  5 � 29.540.000   6 � 29.550.000   7 � 29.560.000   8 � 29.570.000 �

�  9 � 29.580.000  10 � 29.590.000  11 � 29.600.000  12 � 29.610.000 �

� 13 � 29.620.000  14 � 29.630.000  15 � 29.640.000  16 � 29.650.000 �

� 17 � 29.660.000  18 � 29.670.000  19 � 29.680.000  20 � 29.690.000 �

����������������������������������������������������������������������

FREQUENCY LOAD AS A RESULT OF LOADING THE FILE TENRPT20.MEM

**YOU COULD HAVE ALSO TYPED "LOAD TENRPT20.MEM" **

**FROM THE COMMAND LINE TO ACHIEVE THE SAME RESULT!**

* SEE THE SECTION "CAT980 COMMAND SET" FOR OTHER *

* COMMAND LINE SHORT CUTS *

Now let's take a look at what exactly happened. First, you once

again saw the memory display being loaded with some of the 10 meter FM

channels.   There was also another load going on in the CAT980 slots; do

a <^f> (after you 'hit any key' as requested) to view the contents of

the 300 memory slots and note that 20 of the 300 slots are now loaded

with 10 meter FM frequencies.  How did this come about?  The file

FM10MRPT.MEM did it. You can look at this file, it's just plain text

made in an ASCII editor.  You can do the same - write a file to do what

you want it to do.  Appendix B will tell you how to create your own.

*DEFAULT FOR <ShftF1> IS .MNU *

Now do a <ShftF1> again and supply the name 'DEMO20' without

the '' marks.  If you're new to this software get ready to gasp because

this is where some real fun begins!  Use up/down arrows to pick a

-24-

CAT980                                                 Ver 2.0  Page 25

selection to instantly deploy the radio. Run the arrow down to selection

number 16 'CB Channel 19'.  Hit <CR> 'good buddy' and listen to the roar

of undisciplined amplitude modulated madness of the 27.185.000

frequency. (It's like 20 meters only with carriers.)

* <ShftF1> CAN HELP YOU CREATE .MNU OR .LOG FILES ON THE FLY *

* BUT IT WILL NOT CREATE A .MEM FILE *

Can't take it? OK, let's go for another selection - but STOP,

don't bother to use <ShftF1> again. Just hit the <SpaceBar> and the menu

returns. Make a choice, and when you do notice that the screen analog

dial is updated to your choice.  Forgot your choice?  Notice that line

one on the top of your screen displays the menu line you chose.  It will

remain on screen until CAT980 decides it is no longer valid, usually

when you issue a new major command.

This supplied menu can be changed, or better yet new ones

created by you to reflect your operating habits.  See Appendix C.

Related Plain Text Commands:

'Load fname.ext'        The .ext should be .mnu, .mem or .log.

'DIR'                   Lists all files in the CAT980 directory.

************************************************************************

<Shift F2>      'SAVE'          "Set Frequency/Mode Set" - Save Set Up

This will store the present set up - frequency, mode, filter

shapes, et al, into a special memory slot that can be called back at any

time with the <ShiftF3> key.

Related Plain Text Commands:

'REStore'

************************************************************************

<Shift F3>      'REStore'       "Toggle Freq/Mode Set"  Restore Set Up

Saved by <ShftF2>

This toggles between the setup saved by <ShiftF2> and whatever

the configuration was when you hit <ShftF2>.  Let's say you saved a

setup with <ShiftF2> that was USB but you are presently monitoring a TTY

frequency in FSK down the band. You can toggle between the two setups

quickly - seeing if that USB station you are looking for is on the air,

yet dropping back to get some copy from the RTTY station.  One

interesting capability is that you can also use it for a quick USB/LSB

switch to check a pal's unwanted sideband emission if you want to set it

up that way.

Related Plain Text Commands:

'SAVe'

************************************************************************

-25-

CAT980                                                 Ver 2.0  Page 26

<RightShift>    (none)          "The DX'er Key"

This is a special frequency toggle you can use in the split

mode. Example: if you are on 40 meters transmitting on 7.190.00 and

listening for JA's on 7.070.00, this key will give you instant receive

access to 7.190.000 so you can see if that frequency, the one you are

transmitting on, is clear. Another use would be the 10 meter FM repeater

splits so you may listen to the input frequency if you wish. It's

somewhat like a REVERSE button found on many 2 meter rigs.

This feature is also handy in checking another station for

spurious emissions - allowing you to toggle between the unwanted signal

and the QSO frequency quickly for a report.

It only takes effect when you hold the key down.  Let it up and

it returns you to normal! (If you're not in a SPLIT condition then the

key is non responsive.)

************************************************************************

<Shift F4> thru <Shift F8>  "I.F. Filter Control Group"

-------------------------------------------------------

These commands allow the same control of the I.F. filters that

you can get from the active filter display <F8> - only no display need

be active so you always have control of the filters at all times.  The

commands are:

<Shift F4>      'SHIft -xxx'     Shifts the filter up band by 100 hz.

<Shift F5>      'SHIft xxx'    Shifts the filter down band by 100 hz.

<Shift F6>      'WIDth xx %'    Narrows skirt toward lower freq.

<Shift F7>      'WIDth -xx %'   Narrows skirt toward higher freq.

<Shift F8>      'CENter'        Centers the IF filters, width & shift.

NOTE: WHY DOES A NEGATIVE SHIFT GO 'UP BAND' AND A POSITIVE

SHIFT GO 'DOWN BAND' (Why ask Why?)? THAT'S WHAT HAPPENS

FROM THE FRONT PANEL IN MANUAL CONTROL SO CAT980 IS TRYING

(HARD!) TO BE CONSISTENT.

The 'SHIft' commands above may be from 100 to 1000 or -100 to

-1000. The FT-980 will only allow shifts in 100 Hz increments.

The 'WIDth' commands may be from 1 % to 100 % and -1 % to -100

%.  The per cent sign '%' is not necessary but will be recognized by the

program. The actual width change will be as close to your command as the

FT-980 will allow it to be.

-26-

CAT980                                                 Ver 2.0  Page 27

* YOU CAN NOW NARROW THE FILTER BANDWIDTH INSTANTLY *

* WITH ONLY ONE STATEMENT FROM THE COMMAND LINE! *

*(Registered Users Only)*

Related Plain Text Commands:

'CENter filter'         Centers both filter Width and Shift.

'CENter WIDth'          Center filter Width only.

'CENter SHIft'          Center filter Shift only.

************************************************************************

<Shift F9>      'PUT'           "Memory Channel to Operating Mode"

This command activates the selected memory channel in the FT-980

into the VFO mode - something the front panel would never let you do.

This is not the same thing as selecting a memory channel where you will

be 'stuck' with whatever mode and vfo is stored in the FT-980 channel.

After a <ShftF9> you have control of the parameters as if they

had been manually entered. Indeed, that's exactly what CAT980 has done

for you and at maximum speed.  The memory channel activated can be

chosen thru the <F7> menu or a 'CHANnel xx' command. You will be

prompted to determine if you really wish to accomplish this transfer.

* NOW YOU CAN INSTANTLY INSTALL THE PRESENT MEMORY CHANNEL *

* INTO THE VFO WITH A SINGLE STATEMENT FROM THE COMMAND LINE *

Related Plain Text Commands:

'MEM980'        Calls up Active Memory Display (as <F7>).

'SHOw MEMory'   Call up INactive Memory Display.

'GO xx'         Call FT-980 memory channel, xx from 1 to 16.

'CHANnel xx'    Same as 'GO xx'

* NOW YOU CAN INSTANTLY CALL AN FT-980 MEMORY CHANNEL  *

* WITH A SINGLE STATEMENT FROM THE COMMAND LINE *

************************************************************************

<Shift F10>     'ALTernate time'    "Alternate Time Frequency"

This is the same as <F10> only it brings up a special CAT980

channel that you have picked to list in the config file.  You could use

the <ShftF10> key to call up WWV on 15 MHz. Or perhaps you don't want

WWV. Some unique time broadcasts (presented here in CAT980, 7 digit

format for files) are:

Australia:  0542500  0751500  1200500

Canada:     0330000  0733500  1467000

That remark about '7 digit format' will be made clear when you

learn how to write your own menus, memory and config files in the

appendices.

-27-

CAT980                                                 Ver 2.0  Page 28

The default is WWV on 5 Mhz. Many countries use the same

frequencies as WWV so in a sense these are generic time standard

frequencies.  Of course you may choose not to use this key for a time

standard station. You could just as well program your config file to

have <ShftF10> produce 650 Khz(0065000).  It would then become a Grand

Ol' Opera key (WSM in Nashville, Tn.). That's not what it's intended for

but, hey, feel free...(this slot is permanently in AMW).

THE ALTERNATE FUNCTION KEYS

===========================

To produce the following results, hold down the Alternate key

while you push the desired function key.

KEY(s)          Plain Text      Explanation

-----------     ----------      ----------------------------------

<Alt F1>                        Reserved for future versions.

<Alt F2>        none            Toggles screen display between the

previous Ver 1.8/1.9 screen displays and

the new Ver 2.0 series displays.

<Alt F3>        'CAT980'        This toggles the relationships between

'FT980'         the various filters when you change from

(Registered User)               mode to mode. An example might be when

you are in the CW mode  and listening to

a CW signal  - then switch to USB.  The

signal disappears! and that's what it is

supposed to do.  Take a moment to look

at the relationship of the various

filters up on the screen and shown below

on the left. This is the way YAESU(tm)

built the radio. Now look at the display

below right, after an <AltF3>.

NORMAL  | OPERATION                 B F O  | 700 Hz

�����������������Ŀ                   �������Ŀ

� AM/FM  |        �                   �USB    �

�������Ŀ|�������Ŀ                       �������Ŀ

�USB    �|�    LSB�                       �    LSB�

|  ����Ŀ                        ����Ŀ

���ܿ� FSK�                        � FSK�

�CW �                              ���ܿ

�������������������                       �CW �

3 KHz     ^      -3 KHz             �������������������

YAESU Filter Model               3 KHz     ^      -3 KHz

CAT980 Filter Model

-28-

CAT980                                                 Ver 2.0  Page 29

What is the value of this?  Say you are in QSO on USB and

suddenly a station begins to call you on CW.  By doing an AltF3 and

switching to CW mode, you can now answer that station without doing a

wild frequency-signal hunt.  And you can go right back to USB without

losing the original station. You must be on frequency or you do start to

experience a tonal difference. It always helps in this hobby not to be

tone deaf.

<Alt F4>        'FSK'           This toggles between the FT-980 FSK

'AFSK'          tones and an external AFSK device such

as a PK-232. To experiment with this

(Registered User)               function first make sure the FT-980 is

in the YAESU's configuration (see AltF3

above) and go into the FSK Mode (simply

type FSK <RETURN>). Now type AFSK

<RETURN> and the CAT980 AFSK Filter

Model shown above will appear.

You can set the Mark frequency with a statement in the

.cfg file. The default is 2100 which corresponds to the

PK-232. Now if you have a digital signal, say RTTY, and

are set up in the AFSK mode, you can switch to LSB OR

USB and the mark frequency is the same 'distance' from

the operating frequency as it was in FSK. This actually

reverses mark and space from LSB to USB but most

important keeps the signal in the proper position in the

band pass.

M A R K  | 2100 Hz

�������Ŀ

�USB    �

�������Ŀ

�    LSB�

����ܿ

� FSK�

���ſ

�CW �

�������������������

3 KHz     ^      -3 KHz

CAT980 'AFSK' Filter Model

<Alt F5>        'AGC ON'        Toggles the AGC sensing capability of

CAT980. You should have this function on

(Registered User)               if you have installed the circuit in

Appendix G. During scan operations the

AGC line, the 'S' meter level, will be

monitored for a preset level and if that

-29-

CAT980                                                 Ver 2.0  Page 30

level is exceeded an alarm will sound

and the scan will stop. Look at the

display in the upper right hand corner

of the screen. If it shows 'AGC' then

this feature is activated.

'AGC OFF'       When off, the scan functions will behave

in the same manner as they did in

versions   1..8/1.9.

<Alt F6>        none            This toggles the on-screen/off-screen

display of the file box information.

���������������������ķ

� ACTIVE FILES LOADED �

� ------------------- �

� MeNU:   DEMO20.MNU  �

� LOG:      (none)    �

� MEMory: EXAMP20.MEM �

���������������������Ľ

<Alt F7>        'CALibrate'     This brings up a unique display which

allows you to quickly set up any AGC

(NOTE: This has been made       level you wish for the scanning

available to the Public         function.  It can also be used as an

Domain so that you may          installation tool when installing the

experiment!)                    circuit in Appendix G.

������������������������ C A L I B R A T E  AGC ���������������������ͻ

�                                                                     �

� Adjust FT-980 AGC interface circuit for a change of this display.   �

� [COM 1]                                                             �

�           o----------------/\/\/\---o o-GND                         �

�      CTS  |   o GND                 /\                              �

� _____pin_5|___|7_____________ To AGC Interface                      �

� \ o o o o o o o o o o o o o /                                       �

�  \ o o o o o o o o o o o o /  For 25 pin connectors                 �

�   \_______________________/���������������������������������������Ŀ�

�                            �"SIGNAL DETECTED" - CTS PIN IS HIGH (+)��

�           o---o GND        ����������������������������������������ٺ

� _________5|__                                                       �

� \ o o o o o /  For 9 pin connectors                                 �

�  \ o o o o /             To AGC Interface                           �

�   \____|__/  CTS             \/                                     �

�  pin 8 o------------/\/\/\---o o-GND                                �

���������������������������������������������������������������������ͼ

Alternate F7 Calibration Display

<Alt F8> thru <Alt F9>          Reserved for future versions.

-30-

CAT980                                                 Ver 2.0  Page 31

TABS

=====

What we're talking about here is the FT-980 Tab functions not to be

confused with the TAB key on the computer keyboard.  The FT-980 seems to

accept some unacceptable tab limits like the lower frequency being the

upper tab limit.  Worse, if you really want to drive yourself crazy set

up the TABS for 0 frequency and - by golly, it will lock on there if you

command it to (phooey, don't do it!).  In other words they can be set

wrong if care is not taken.  CAT980 will help keep you honest by

discouraging zero frequency tabs when it detects such a situation.  But

it's a good idea when setting tabs to use the same precautions outlined

in YAESU the radio manual. Common sense will keep CAT980 and FT-980

happy.

CAT980 uses a lot of Control Key combinations.  As you know this

means you hold the Control (CTRL) key down while you press a 2nd key.

The up-carrot convention, ^, will be used in this text to mean hold down

the Ctrl key.  So a <Control L> will be shown as <^l>.

<^l>    'SET Lower tab'         Set Lower Tab Limit. Whatever the

operating frequency is when a <^l> is

executed will be the lower tab limit.

<^u>    'SET Upper tab'         Set Upper Tab Limit. Whatever the

operating frequency is when a <^u> is

executed will be the upper tab limit.

<^t>    'TABs'                  Toggles the radio tab function on and

off.

<^k>    'AUTO tabs'             Toggle automatic Tab Limits. This means

new tab limits will automatically be

switched in when changing bands. See

Appendix D.

<^w>    'RESET Tabs'            Resets ALL TABS to CAT980 defaults.

'CLEAR Tabs'            Same as above.

Below is an example procedure to set the tabs to the 10 meter

novice phone band:

1. To Set lower tab.

<F2> Key in 28.300.000 as the operating frequency.

<^l> set lower tab.

-31-

CAT980                                                 Ver 2.0  Page 32

2. To Set upper tab.

<F2> Key in 28.500.000 as the operating frequency.

<^u> set upper tab.

3. To Activate tabs.

<^t> This command toggles tabs on and off.  When on,

notice that the <Home><End> keys now define the band

limits as 28.3 to 28.5 Mhz..  Try a band scan <^b> with

the tabs activated.

One last note on tabs.  The radio itself stores two complete

sets, one for GEN and one for HAM operation.  It's good practice to have

both preset to sensible limits before operation is switched to CAT980.

Doing a <^q> will display the state of the present tab settings. To set

the tabs properly (outside of CAT980) see the FT-980 manual.

Appendix A and the cat980.cfg file itself are aids to setting

the TAB defaults to values other than U.S. band limits.  You can

configure your system to come up in Novice bands or perhaps limits

required by other governments. Exciting menus can be created using the

tab feature. Study the included file SWL20.MNU to see this in action.

Related Plain Text Commands:

'TABs ON'

'TABs OFF'

'AUTO tabs ON'

'AUTO tabs OFF'

************************************************************************

-32-

CAT980                                                 Ver 2.0  Page 33

CHAPTER FOUR - SCANNING OPERATIONS

�������Scan Options�������ͻ

� Scan FT-980 Memories     �

� Scan Band                �

� Scan Marked Frequencies  �

� Scan Menu Selections     �

��������������������������ͼ

The <F9> Menu Display

Enter the scanning operations with <F9> or type 'SCan' for a

menu, or choose a direct command listed below to enter a particular

scanning mode.  There are four main scan possibilities and we'll look at

each of them one at a time.

* FOR REAL SCANNING FUN BUILD THE SIMPLE INTERFACE IN APPENDIX G *

* IT USES THE FT-980 AGC LINE AND THE COMPUTER CTS LINE FOR CONTROL *

To activate the AGC feature type 'AGC' or use <AltF5>.  See

Appendix G for instructions on how to calibrate your system. Of course

the scanning functions work without the AGC line hook up, but it

requires manual intervention to stop a scan as in earlier versions.  The

ability of the program to sense a preset 'S' level for you and to act

upon it is one of the main features of Version 2.0 - so consider

building the simple circuit in Appendix G.

In the following discussion the difference in behavior between

AGC and NO AGC will be shown when applicable. In general it will revolve

around the 'Scan Hang Time' parameter. You can toggle the AGC sensing

function on and off with <AltF5>.

�����������������������������������������������������������������ͻ

�                      NO AGC SENSING                             �

�                      --------------                             �

�When  you're  prompted for the 'Scan Hang Time' you may enter    �

�that period that the FT-980 will remain on a frequency or        �

�channel before it moves on to the next one. The hang default is  �

�1 second.  This parameter may be entered in tenths of a second   �

�up to 299 seconds.  So .2, 2 and 200 are all valid entries for   �

�200 msec, 2 seconds or  200 seconds respectively.                �

�                                                                 �

�                                                                 �

�Since the AGC line is not being monitored the scan will simply   �

�continue to scan and stop only as long as you have programmed    �

�it to with this parameter so it's your reflexes may dictate      �

�this number.  It is up to the operator to stop the scan when     �

�desired. It's your call; when you hear a station or whatever,    �

�hit any key to halt the scan.  You will be prompted with other   �

�options at that time.                                            �

�                                                                 �

�����������������������������������������������������������������ͼ

-33-

CAT980                                                 Ver 2.0  Page 34

�����������������������������������������������������������������ͻ

�            AGC SENSING - 'THE ONLY WAY TO GO!!'                 �

�            ------------------------------------                 �

�                                                                 �

�When  you're  prompted for the 'Scan Hang Time' with the AGC     �

�line being monitored, you will enter the same parameter as       �

�described above.  But the important difference is that you       �

�don't have to listen to the signal as CAT980 does that for you.  �

�It samples the state of the AGC line and then remains on a       �

�channel if and when it detects a signal. If the program          �

�determines that the 'S' meter level you have preset (See         �

�Appendix G) has reached the predetermined level you have set     �

�during  calibration,  it  will halt the scan automatically and   �

�remain on  that  frequency  for approximately 5 seconds to give  �

�you the opportunity to stop the scan entirely if you so          �

�choose.. After these 5 seconds it will begin the scan again      �

�unless the operator has indeed intervened by hitting a key.  If  �

�no signal is detected than the scan moves on at the hang time    �

�speed which can be rather quickly (system dependant). While you  �

�can enter other time increments it is recommended that you       �

�choose NORM because this has been determined as the max scan     �

�speed for you already. More time control will most likely be     �

�available to you in later versions - after you send in your      �

�comments and suggestions.                                        �

�                                                                 �

�If a high enough AGC level is detected than CAT980 will make a   �

�game-type noise and change colors and display the word           �

�'SIGNAL!' to alert you that a signal has been detected. If you   �

�hit a key at this point you will be prompted about other         �

�options.                                                         �

�����������������������������������������������������������������ͼ

-[Registered Users]-

Choosing high numbers for the scan hang time is possible.

Two-ninety-nine seconds, for instance,  is about 5 minutes and may seem

a bit excessive for most operating.  On the other hand one may be

scanning fax frequencies and time is needed to present part of the

picture on the screen. Then there's a reason you may have for such a

long scan that hasn't occurred to this op. yet. Also keep in mind that

time-wise you're limited by the 4800 baud rate of the FT-980 and your

system speed.

<^n>   'SCan MEMory'   SCAN THE FT-980 CHANNELS - The 1st Menu Choice

---------------------------------------------------------------------

The first choice from <F9> is 'Scan FT-980 Memories' which you

choose with <CR> from the menu or you could have used a Control N (^n)

from the command line - or typed 'SCan MEMory'.

-34-

CAT980                                                 Ver 2.0  Page 35

��������������������������������������������ͻ

�FT-980 I N T E R N A L  M E M O R Y  S C A N�

�                                            �

�     Scan Chn 1 to Chn:  _       <CR>=16    �

�                                            �

�                                            �

��������������������������������������������ͼ

You're first prompted for how many channels to scan, from 2 to

16.  A <CR> defaults to all 16 channels.  Next you're asked about the

timing which is covered above.

* AGC OR NOT *

* THE <ESC> KEY ABORTS ALL SCANS *

OK, WITH OR WITHOUT AGC sensing you stop the scan by hitting any

key. In AGC the scan is halted for about 5 seconds ('NORM') if a signal

is detected so that you have plenty of time to evaluate what you hear

and hit a key to keep the program from advancing to the next channel -

or ignore it and let the scan begin again by itself.

The channels being scanned in this mode, of course, are those of

the FT-980 memory bank. These will have been set up before hand.  You

can also do it from the keyboard using the Insert <Ins> key (write-to-

memory) - or better yet, load the whole bank quickly with a memory file

(see ShftF1). If neither of these are done then the memory bank will

contain the same set up as when CAT980 is not in control - before you

brought up CAT980 and it took control.

Try it. 1)Do an <F9> to bring up the menu. 2)Do a <CR> to pick

the memory scan. 3)Do a <CR> to pick the 1 second default. Note that the

channel number, the frequency and the mode are all sequentially

displayed.  When you get tired of the scan or hear something you want to

investigate, hit a <CR>.

The command line now shows you the following options:

Any=resume scan | BkSp/^BkSp=last | ESC to exit

So, any key will resume the scan, a <BackSpace> will put you

back one channel, a <CtrlBackSpace> will advance you one channel and an

<ESC> will abort the scan.  By the way, during the scan itself an <ESC>

would also abort the scan. At this time hit the <ESC> key.

Notice what happens. The <F7> Radio Memories Active Display

automatically pops up in it's menu mode.  You may now install the

channel you stopped the scan on, into the VFO or simply pick the FT-980

memory channel itself.

-35-

CAT980                                                 Ver 2.0  Page 36

<^b>    'SCan BAnd'     BAND SCAN - The 2nd Menu Choice

-------------------------------------------------------

This scan mode activated by a <CR> from the <F9> menu's 2nd line

or a Control B <^b> - or type 'SCan BAnd' from the command line - offers

a number of interesting possibilities.  First the end limits of the band

default to the ham band edges - if HAM VFO selected - OR you can set the

limits by setting the Tabs (<^l>,<^u>,<^t>) which may also be

accomplished by a menu selection - OR you can do an auto-tab control

<^k> - OR - better see Appendix D for the tab controls!

Before you try this feature get into the HAM VFO on one of the

ham bands. First type 'VFO' <CR>, then 'HAM' <CR>.  This might be a fun

place to try another feature, type 'TWELve meters' - and ZOOM - you are

mid band on twelve! The other bands may be quickly called the same way.

(Do you know the band edges for 12 meters by heart? CAT980 does!)

Now do a <^b> and accept the 1 second default - or NORM if AGC

is on - with a <CR>. (You could have picked other time limits as

described above.)

Stop! Here's something new. You are being prompted for the

frequency differential in KHz, that 'distance' from one scan frequency

point to the next. The default is 10 Khz or the last differential

entered using the <+> or <-> commands (described later) - but you can

choose any difference frequency from 10 hz (enter 0.001) to 500 Khz

(enter 500).  Pick a reasonable number in relation to the band limits.

For now accept the 10 KHz default with a <CR> or enter 10 <CR>.

Watch the analog pointer moving across the band. Glance at the

frequency display on the FT-980 and notice that it too is following the

scan.

Back to the subject of picking a reasonable frequency distance

between scan points.  For instance, if CW is of interest one might

choose 100 or 200 hz and perhaps 2 Khz for ssb. CW signal should be at

least 200 Hz apart and SSB signals 2.7 KHz apart (don't we wish!).

When a band edge is detected by CAT980 the frequency will jump

to the other edge and continue the scan.  Once the scan begins you can

stop it by hitting any key - even if the AGC feature is activated.  Do

it; now you have some new options.  You can:

1) Use the back space <BkSp> to move back down thru

the scan frequency points (10 KHz each).

2) Use the Control back space <CtrlBkSp> to move forward

thru the scan frequency points (10 KHz each).

3) Hit a <CR> to resume the scan.

4) Hit a <ESC> to stop and abort the scan.

-36-

CAT980                                                 Ver 2.0  Page 37

<^v>    'SCan SLOts'     SCAN MARKED FREQUENCIES - The Third Menu Choice

------------------------------------------------------------------------

This mode will scan the frequencies you have marked during

previous operation with a <^s> or loaded from a .mem file.  There must

be at least 2 such frequencies in the slots before the scan command is

honored and there can be as many as 300. (See Appendix B.)  You will be

prompted for a 'hang time' discussed above.

Once the scan begins simply hit any key to stop it. By the

way, unlike early versions of the program you can interrupt a long hang

time like 5 minutes. If you have a fast scan in progress and you don't

hit a key fast enough and overshoot the frequency of interest, use

<BkSp> and <^BkSp> to back down or go forward thru the frequencies.  A

<CR> will start the scan again from the any slot or an <ESC> will exit

the scan leaving the last frequency as the operating frequency.  If you

are scanning with the AGC feature the scan will of course stop when it

detects a signal - but if you take no action, don't hit a key, it will

resume the scan automatically.

* PSEUDO SLOTS ARE STORED FREQUENCIES WHICH ARE BEING IGNORED *

* DURING A SCAN SIMPLY BECAUSE THE RADIO CAN NOT DELIVER *

Did you happen to notice some 'strange' things going on when you did

this scan?  If you were set up right, VFO HAM 12 meters, than you

experienced what CAT980 calls 'pseudo' slots.  That is a slot that is in

memory but can not be accessed because the FT-980 wasn't set up for it.

Assuming you had examp20.mem loaded here's what actually happened:

1. The scan began on the highest slot, # 232 which is 29.6 MHz

and began a sweep down. The FT-980 being in VFO and HAM allowed the

sweep down - if you look over at the radio you will see it changing

frequency as ordered.

2. BUT - when it got to slot #216 it stopped, the radio stopped

on 28 MHz - it's in HAM VFO remember, so it can't go any lower, it's at

a physical limit.  That's when the command line showed you that 'pseudo

slots' where being scanned - or, actually, NOTHING was being scanned.

3. Since this is all happening above 12 meters, the display you

have on the screen, the left blinking arrows indicate that what is going

on is above twelve meters the weird character that has replaced the

usual dial pointer means that no frequency change is happening.

4. BUT if you are patient for a moment you will see the scan

come rolling by and the radio begin to respond again. This happens at

slot #185, 24.9 MHz - and then off to the right and down the band it

goes showing a blinking right arrow and then the weird dial pointer

meaning it's out of the HAM VFO range again.

5. It will continue to 'scan' pseudo slots until it enters

another ham band, fifteen meters.

-37-

CAT980                                                 Ver 2.0  Page 38

You don't like pseudo slots?  Well, if the radio vfo was in GEN

than you won't be bothered by them.  Try the same scan after you do a

GEN <CR> or hit <F4>. Now the ordered scan is possible because the

frequencies lie withing the physical limits of the radio.

<^m>    'SCan MENu'     SCAN MENU SELECTIONS - The Fourth Menu Choice

---------------------------------------------------------------------

This is a brand new scanning option in the Version 2 dot series.

You bring it up from <F9> by choosing the fourth option - or do a <^m> -

or type 'SCAn MENu' <CR>.  As you may know a <^m> is equivalent to

<RETURN> which you may use to start the menu scanning action.

The only pre scan query will be the one dealing with timing.

The information on timing above applies, but there is another factor to

consider here and that is a menu scan by it's very nature can not be a

fast scan - even if you have a very fast PC. A menu selection may change

a number of FT-980 parameters: frequency, mode, filter widths and shifts

- all of these must be in tandem at 4800 baud.  But in this op's opinion

it is 'fast' enough.  Choose 'NORM' if you're in AGC mode.

As the menu is scanned notice that each plain text line you have

written for a given menu selection is displayed at the top of the

screen.  CAT980 is hopefully smart enough to recognize a non-scannable

selection - like you may have one that loads a .mem file or

calls/returns to another menu.  When the scan detects such an entry it

will ignore it and go on to the next selection.

Notice that the demonstration menu skips selections 9, 14, 15

and 21.  '[9] QRP Frequency Menu' is number 9 and if you examine the

others you will see that they are of a similar nature.

One almost exception is selection 19: '[19] Novice Bands'. At

first glance you might think this calls a menu.  When you study the file

you will see that a number of tabs are set defining the novice band

edges.  These are ignored during the scan - but the embedded frequency

and mode commands are legitimate to the scanner so it will stop and

check 3.71 MHz in CWW.

Like the other scans you can halt the action by hitting any key

or the AGC will stop-and-dwell on a detected signal.  When you do hit a

key you can step forward or backward through the menu selections with

<CtrlBackSpace> and <BackSpace>.

If you'll think about this new feature for a bit you should be

able to come up with some unique scanning ideas to create new menus.

The display of your plain text info should be a big plus in special type

operations.  How about a MARS menu scan?

-38-

CAT980                                                 Ver 2.0  Page 39

MANUAL SCAN - Not On The Menu But Real Enough

---------------------------------------------

We opened this portion of the documentation by saying that there

are four scanning modes.  We should have said five.  Actually this is

not a scanning option as such but is a byproduct option of the frequency

commands <-> and <+> which can be used most effectively in some

situations.  Simply use the commands, <+> and <-> keys, to 'walk' up or

down a spectrum. Once you have selected the amount, say 2 Khz, it is

only necessary to manually keep double-hitting the key to produce a scan

type effect, but try not to overload your computer buffer by holding

down the key indefinitely (each computer is different in this respect).

Manual scan can be from 10 hz (enter 0.01) to 500 Khz (enter 500)

increments using the <+><-> keys.

A manual scan can also be had with the <BkSp> and <^BkSp> keys

which will move you through the CAT980 loaded slots. This is available

even when you are not in a scan mode - but only if slots have been

loaded. If all slots have been cleared, <^d>, then <BkSp> and <^BkSp>

will walk you through the active menu selections.

-39-

CAT980                                                 Ver 2.0  Page 40

CHAPTER FIVE - CUSTOMIZING YOUR OPERATING FILES

MENU BUILDING - EDITORS & A SPECIAL UTILITY

===========================================

USER MENU'S - YOUR PERSONALITY

------------------------------

You can effectively 'program' CAT980 to suit your operating

needs. It's this capability that allows you to customize CAT980 to your

world of operating from special shortwave broadcast stations to listing

packet frequencies to having quick control of MARS operations.  You will

create an unlimited number of operating menus in plain text ASCII

editors and CAT980 will read them in, display them, and deploy the radio

to your menu selection choice. Like snapping your fingers.

* AFTER YOU CHOOSE A MENU SELECTION *

* CAT980 DISPLAYS YOUR PLAIN TEXT DESCRIPTION AT THE TOP OF THE SCREEN *

Menus can even be nested. That is, menus can call menus and

return to master menus.  We've already played with this somewhat. Hit

the <spacebar> to bring up DEMO20.MNU again.  If for some reason demo20

doesn't appear, than you can bring it up with <ShiftF1> (see above).

Look at selection number 9: 'QRP Frequency Menu'.  Bring the

cursor down with the arrow keys or better yet, simply type <9><CR>.

Here's what you see on the screen:

��������������Q R P  F R E Q U E N C I E S  (per QST Feb. '90)�����Ŀ

�[1] CW - 160 Meters  - 1.810                                       �

�[2] CW - 80 Meters - 3.560 MHz                                     �

�[3] SSB (LSB) 75 Meters  3.985 MHz                                 �

�[4] SSB (LSB) 40 Meters  7.285 MHz                                 �

�[5] CW 30 Meters  10.106 MHz                                       �

�[6] CW 20 Meters 14.060 MHz                                        �

�[7] CW 15 Meters 21.060 MHz                                        �

�[8] CW 12 Meters  24.9 MHz                                         �

�[9] CW 10 Meters  28.060 MHz                                       �

�[10] SSB (USB) 10 Meters  28.885 MHz                               �

�[11] Return to DEMO20 Menu                                         �

���������������������������������������������������������������������

-40-

CAT980                                                 Ver 2.0  Page 41

The file qrp20.mnu that was written to produce this screen is as

follows:

----------------------------qrp20.mnu ---------------------------------

title=Q R P  F R E Q U E N C I E S  (per QST Feb. '90)

menuCOLOR=K,w

[1] CW - 160 Meters  - 1.810

FREQ=0181000  VFO=HAM CONTROL=VFO MODE=CWW

[2] CW - 80 Meters - 3.560 MHz

FREQ=0356000  VFO=HAM CONTROL=VFO MODE=CWW

[3] SSB (LSB) 75 Meters  3.985 MHz

FREQ=0398500  VFO=HAM CONTROL=VFO MODE=LSB

[4] SSB (LSB) 40 Meters  7.285 MHz

FREQ=0728500  VFO=HAM CONTROL=VFO MODE=LSB

[5] CW 30 Meters  10.106 MHz

FREQ=1010600  VFO=HAM CONTROL=VFO MODE=CWW

[6] CW 20 Meters 14.060 MHz

FREQ=1406000  VFO=HAM CONTROL=VFO MODE=CWW

[7] CW 15 Meters 21.060 MHz

FREQ=2106000  VFO=HAM CONTROL=VFO MODE=CWW

[8] CW 12 Meters  24.9 MHz

FREQ=2490000  VFO=HAM CONTROL=VFO MODE=CWW

[9] CW 10 Meters  28.060 MHz

FREQ=2806000  VFO=HAM CONTROL=VFO MODE=CWW

[10] SSB (USB) 10 Meters  28.885 MHz

FREQ=2888500  VFO=HAM CONTROL=VFO MODE=USB

[11] Return to DEMO20 Menu

load=demo20.mnu{end}

------------------------qrp20.mnu end file-----------------------------

Notice that the second line of each selection is the 'business'

line. It's the one with the FT-980 parameters that are fed to the radio

when a selection is made. And there could have been a lot more on this

line.   Your menu entries can call out FSK shift, filter width, filter

shift - and even split frequency operation for such operating as 10

meter FM repeaters. And you can even color code your menus.  How do you

do it?  See Appendix C for details on how to create your own menus.

Appendix E addresses coloring.

-41-

CAT980                                                 Ver 2.0  Page 42

You can type up an unlimited number of these files a character

at a time.  Ah, but there's an even better way to construct menus - do

it on the fly in real time!  Before we learn about it let's return to

the DEMO20 menu from the QRP menu - you do that by picking selection 11.

(<SpaceBar><11><CR>).

And before we go on just a reminder; you could have brought up

the demo20 menu from the command line by typing 'load demo20.mnu' <CR>.

OK, drop the menu off of the screen by hitting <SpaceBar>.

THE MENU BUILDING UTILITY - Let Your Computer Write Your Menus!

===============================================================

What happens when you come across a 'hot' frequency you

would like to have in a menu?  You spend time tweaking the frequency and

filters, you get everything just the way you want it.  Do you have to

write down all these variables so you can add them as a second line, the

business line, to your menu -  later in an editor?  Risk losing time and

even data?  NO; just hit <AltP>!

<AltP>     'MENU'   The Pop Up CONSTRUCT MENU UTILITY

------------------------------------------------------

With this pop up utility you can work on your menu in run-time

with ease.  Do an Alternate P <AltP> - or type 'MENU' <CR> from the

command line -  and notice that when you call the utility it comes up

with all FT-980 parameters instantly supplied for you.  Frequency, mode,

control and vfo are displayed and in the proper CAT98O format and ready

to go to disk and/or a printer if you so choose.  If shift and/or width

are not centered then these elements will also be supplied as well as

the FSK shift frequency if in FSK mode. When you do a <AltP> you will

get a display something like this:

�������������������� CONSTRUCT MENU UTILITY �������������������������ͻ

�Active Menu: cat980.mnu                                              �

�[5]__________________[type up to 70 characters if desired]___________�

�                                                                     �

�FREQ=1410745 VFO=HAM CONTROL=VFO MODE=FSK FSK=170 IFSHIFT=-0400      �

���������������������������������������������������������������������ͼ

Did you notice that DEMO20.MNU was first selected as the active

menu - but then CAT980 rejected it and changed the name to CAT980.MNU?

That's because DEMO20.MNU was full, it already has 21 selections.  If it

had less than 21 it would have remained as the active menu and allowed

you to continue to add to it.

-42-

CAT980                                                 Ver 2.0  Page 43

And notice that the menu selection number is also supplied!  It

will always be the next unused selection up through 21. (On the included

file in this package that will be selection [5] the first time you call

it.)  Now you supply the text which you want to display following the

supplied selection number when the menu is called.  After you do that

and hit a <CR> you will see the following:

����������������������� CONSTRUCT MENU UTILITY ����������������������ͻ

�Active Menu: cat980.mnu  ESC=ABORT | F1=PRINTER | F2=DISK | F3/CR=ALL�

�[5] This is the line of text you supplied to describe this selection.�

�                                                                     �

�FREQ=1410745 VFO=HAM CONTROL=VFO MODE=FSK FSK=170 IFSHIFT=-0400      �

���������������������������������������������������������������������ͼ

You may now abort, print, save to disk or print and save to

disk. Again, note that the radio commands are displayed and stored for

you. F2 will be your most probable choice as this key alone will build

the menu file called cat980.mnu.  CAT980.MNU will then be available

again as the default menu building file.

When the menu is full, that is you have created selection [21],

then the utility will ask you to rename the file. Name it a valid DOS

name - and stick with the extension .mnu.

The menu file you will be creating is always that active menu

file which was previously loaded ('MENUFILE=') by the .cfg or a .mnu

file or loaded by <ShftFl> - or loaded by the 'load' command, otherwise

the default cat980.mnu will be supplied for you.  See Appendix A. Now

drop this utility with an <ESC> because we're going to experiment.

Suppose you want to start a whole new activity, suppose you want

to create a menu for forty meters cw.  Pick a name; how about

forty_cw.mnu.  OK, try to load that file.  Type 'load forty_cw.mnu' and

observe the prompt:

Do You Wish To Create FORTY_CW.MNU ?[Y/n]

Let's don't do it yet; answer <n> and note the message.  Now

let's set up for forty cw.  Do the following:

1. Type 'HAM' <CR>. (If you get error, type 'VFO' 1st).

2. Type 'FORTY' <CR>.

3. Type 'CW' <CR>.

4. Type 'SHOW FILTER' <CR>.

5. Type 'SHIFT 100' <CR>.

6. Type 'WIDTH 40%' <CR>.

Now once again type 'load forty_cw.mnu' but this time answer the

prompt <Y>. (You could have answered 'Y' or <CR>, or 'n' to back out.)

But your 'Y' response has the file and ready for your first selection.

(If you don't use [1] and abort with an <ESC> then the file will NOT be

created.) Type some plain text like 'CW sked with K5YEF...' and do a

<CR>. (10 wpm pse)

-43-

CAT980                                                 Ver 2.0  Page 44

Now you have three options: F1 to print, F2 to write to disk,

and F3 does both functions.  Do <F2> and the file will be updated and

the utility dropped.

Drop the filter display off of the screen with <ESC>. Note that

the file display box now shows forty_cw.mnu as the active menu. Hit a

<SpaceBar> and up comes your one-selection menu!

<AltY>     'SWAP MENU' Swap between Active Menu File & Cat980.mnu

-----------------------------------------------------------------

To get back to the original menu from the menu that you are

building use <AltY> or type 'SWAP MENU'.  In general this swap feature

behaves as follows:

1. If you have not loaded another menu that has less than 21

selections you will be given a message that no other menu is available.

2. If the Active Menu is full (21 selections) such as

DEMO20.MNU, then the program will automatically choose cat980.mnu and

inform you with a message.

3. When cat980.mnu itself becomes full (21 selections) then the

program will prompt you to rename it. It is always suggested that you

use names with the .mnu extension for menus.  After you rename the menu

the program will then display selection [1] of a new cat980.mnu ready

for building. You can still choose the Active Menu with <AltY> provided

it is not full and is presently loaded.

THE LOG UTILITY - Uses Your Computer Too!

=========================================

When we talked about creating menus we posed the problem of

what happens when the operator comes upon a 'hot' frequency he or she

wishes to use or make note of.  When you log, is it necessary to write

down all the information of interest such as frequency, mode, filter

shifts, time, date, etc?  Nope; CAT980 can do it for you.

<^p>    'LOG'    The Pop Up NOTE PAD LOG: A Scratch Pad Log

----------------------------------------------------------

This is a handy way to keep operating notes with a lot of info

automatically supplied for you by the program.  When you call the Note

Pad Log the time and date are displayed on the screen ready to be sent

to disk, to print out as hard copy or both. And when you do make a log

entry all the FT-980 parameters are entered automatically - frequency,

mode, filter info - the works.  Right now this is being referred to as a

'scratch pad log' simply because it doesn't have search capabilities,

dupe recognitions - all the features the big boys put into log files.

But someday - maybe?

-44-

CAT980                                                 Ver 2.0  Page 45

When you study Appendix A on writing .cfg files, you will learn

about time stamp options.  In this case if you have used the statement

ZSTAMP=YES in the .cfg file, then the time/date will be in UTC with a

'Z' displayed to let you know. If your logging doesn't require time and

date stamps then you could use the statement TIMESTAMP=NO in the .cfg

file.

And it isn't just the .cfg file that can cause this to happen;

you can toggle the time/date option with .mem and .mnu files too. This

means you could have some logs that deal with local time and some that

appear with Zulu info.

So TIMESTAMP=YES will turn it back on and ZSTAMP=NO will bring

the time display back to local time/date. When you do a <^p> (not to be

confused with the pick-a-port ^p at the very start of the program which

is no longer available after the FT-980 is captured) - or type 'LOG'

<CR> - you will call up a log file.  The name of the file can be

declared in the .cfg file - OR in .mem and .mnu files. (See Appendix F.)

See the implications?  You can associate a log file with a give menu or

by loading a .mem file, even scan activity! (Default is CAT980.LOG). OR

you can bring in a log file with <ShftF1> or the 'load' statement!

When you call the note pad log up you will get a window into the

file that looks something like this:

����������������������� N O T E  P A D  L O G ���������������������ͻ

�16:26     4-AUG-91       Enter up to 254 characters - CR when done �

�                                                                   �

�                                                                   �

�                                                                   �

�FREQ=1640820 VFO=GEN CONTROL=VFO MODE=USB IFSHIFT=-0400 IFWIDTH=076�

�������������������������������������������������������������������ͼ

You can now write 3 or 4 lines of text (up to 254 characters)

with a word wrap feature - that is, you should not use a <CR> until your

text is complete, CAT980 will keep your words and letters together for

you and even supply carriage returns when needed - you just keep typin'.

����������������������� N O T E  P A D  L O G ���������������������ͻ

�16:26     4-AUG-91     ESC=ABORT | F1=PRINTER | F2=DISK | F3/CR=ALL�

�Here's where you type your info - whatever it is you want to file  �

�here, these are your words - and with a word wrap feature too!     �

�Backspace let's you correct mistakes. When done, hit a <CR>.       �

�                                                                   �

�FREQ=1640820 VFO=GEN CONTROL=VFO MODE=USB IFSHIFT=-0400 IFWIDTH=076�

�������������������������������������������������������������������ͼ

Note that on the bottom of the display window the state of the

FT-980 is displayed for you (in CAT980 file format!). These parameters

are saved automatically as part of your log entry. Just like the Menu

Building Utility you can now do one of 4 things:

-45-

CAT980                                                 Ver 2.0  Page 46

1. <ESC> A complete abort - window will drop out, nothing is

recorded.

2. <F1> The window info will be sent to the printer via LPT1.

3. <F2> The window info will be put on disk in the file you have

selected for the present log file. Default is cat980.log; see

<^y> below.

4. <F3> or <CR>. When you do this the info in the window will go

both to the printer and the disk file.

NOTE: IF YOU HAVE CHOSEN A PRINT COMMAND AND THERE IS NO PRINTER HOOKED

UP. - BE PATIENT! IT MAY TAKE A WHILE BUT CAT980 SHOULD RECOVER FROM THE

ERROR. GET ANOTHER CUP OF COFFEE AND HIT NO OTHER KEYS AS YOU WAIT IT

OUT!

<^y>       'SWAP LOG'      Swap between Active Log File & Cat980.log

---------------------------------------------------------------------

The default will always be cat980.log. The log file you have

defined the .cfg or in a .mnu file header will be available as the

Active Log File.

USER MEMORY PROGRAMING

=======================

The FT-980 Sixteen Memories

----------------------------

You may also create an unlimited number of plain text ASCII

files (they must have the extension .mem) to load the FT-980 memories -

any or all 16 of them - when you first enter CAT980 or any time during

operation.  There are 16 memories available even though the radio only

allows 12 from the front panel. (Hmmm, I think that's been pointed out

before.)

OK, try loading the memory file tenrpt20.mem.  Here's how:

1. Hit <ShiftF1>.

2. Type 'load tenrpt20.mem' <CR>.

3. Watch the action on the screen.

What happened? The first 9 memories of the FT-980 were loaded

with 10 meter repeater frequencies (or close enough). Then 20 of these

frequencies were loaded in the CAT980 slots.  And notice that the color

of the display changed!  The file that did this follows:

-46-

CAT980                                                 Ver 2.0  Page 47

-------------------------------tenrpt20.mem---------------------------

* tenrpt20.mem  5/9/92

* (See QST Apr 1990, p 70 'FM/RPT')

* First let's load the 4 main 10 meter repeater output frequencies

* - this will allow us to scan <^n> the memories to listen for repeater

* activity.

Channel=1 Freq=2962000  Mode=fm vfo=HAM

Channel=2 Freq=2964000  Mode=fm vfo=HAM

Channel=3 Freq=2966000  Mode=fm vfo=HAM

Channel=4 Freq=2968000  Mode=fm vfo=HAM

* But if we're even more inquisitive why not go ahead and load the

repeater

* input frequencies as well - if we want we can scan them as well!

* Note that 5 thru 8 correspond to 1 thru 4 only -100 KHz

Channel=5 Freq=2952000  Mode=fm vfo=HAM

Channel=6 Freq=2954000  Mode=fm vfo=HAM

Channel=7 Freq=2956000  Mode=fm vfo=HAM

Channel=8 Freq=2958000  Mode=fm vfo=HAM

* What about the simplex calling frequency? Why not?

Channel=9 Freq=2960000 mode=fm vfo=ham

* No sense limiting ourselves to just FT-980 memory operation - let's

* load the slots for more capability - like a <^v> scan. The band plan

* is compatible with our high speed slot loading feature.

* Be sure to switch to FM before you do a scan...

ClearSlots=YES

Slotxx=2950000/01000/020 *load every 10 Khz for a big sweep

* What else? Why not use the SetTab= feature to define the limits of

* repeater activity and then you can do a band scan <^b>. I'll leave

* this up to you; read the file tab.doc for help.

* Other statements are allowed inside this file. For instance

BOX2=k,c

Bell=no

HamDial=R

-47-

CAT980                                                 Ver 2.0  Page 48

-----------------------tenrpt20.mem continued-------------------------

* Why would you want to change colors? Perhaps certain color

combinations

* could be associated with specific memory and/or slot configurations.

* A glance at the screen colors might tell you instantly that you are

* configured for 10 meter FM repeater activity. This might be done best,

* however, from the menu that calls this memory file.

----------------------(end-file tenrpt20.mem)-------------------------

And that's the way it looks. As you can probably guess lines

beginning with an asterisk * are comment lines.  Don't worry (yet) about

what you don't understand.  Appendix B will shed the blinding light (or

at least a glow) and you can write, write and write your own.

And one point revisited: this memory file could just as well

have been loaded automatically by a menu file selection!  Load up the

menu file DEMO20.MNU and then pick selection 21. It loads examp20.mem

automatically without any action from your own memory files. Here's the

selection lifted from demo20.mnu:

[21] Load and execute memory file examp20.mem

load=examp20.mem

* IMPORTANT: AFTER YOU LOAD A .MEM FILE YOU CAN RETURN TO THE *

* PREVIOUS SET UP BY TYPING 'RESTORE' OR <ShftF3> *

The CAT980 Three Hundred Memories

---------------------------------

Three hundred available memories may seem excessive for most

operations, yet this flexibility offers the imaginative operator his or

her own horizons and a chance to implement some new ideas.  For instance

the memories can be configured to allow a scan of a number of ham bands

to monitor for band openings.  A user memory load file can be created to

scan the spectrum from 30 Mhz down, allowing the operator to spot the

approximate MUF vicinity.  While Appendix B addresses the creation of

these file, you can actually do memory loads straight from the keyboard

as you're operating.  Here are some of the commands and their effect:

<^s>    'MArk'          There are a number of ways to program each and

all of the 300 slots.  During regular operation

a Control S <^s> from the keyboard will load a

single slot and advance the storage pointer

ready to load the next slot. The frequency

stored is the frequency you are on when enter

<^s>.

One use for this feature might be to install a scan to

check for SWR across a desired band.  It goes without saying

that to do this with the transmitter on at high (and low?) power

-48-

CAT980                                                 Ver 2.0  Page 49

or when frequencies are active is not the thing to do.  It's bad

enough out there!  But perhaps when the band is 'dead' and you

use flea power this might be an effective and interesting tool.

(Yes, it will scan in TX!)

<^d>    'CLEAR Slots'   A Control D delete ALL the frequencies stored in

ALL the slots.  You will be prompted to see if

it really is your intention to purge the memory

bank. Y for yes, N for no.

<^f>    'SHOw SLOts'    Display all slots.  This was demonstrated early

in the documentation.  Any frequency shown in

reverse video indicated which slot the program

is parked on.  For instance you might have

stopped in the middle of a scan. <^f> will show

you what frequency that is - or was, if you look

an hour later.

<^a>    'SPEEd slots'   Control A gives you the ability to load the

CAT980 300 memory slots directly from the

keyboard.  The first slot will contain the

present operating frequency and each subsequent

slot will be removed by an amount which you

specify from 10 hz to 100 KHz.  A detailed

overview of this feature is discussed next.

THE 'HIGH SPEED' MEMORY LOAD       'SPEEd slots'     <^a>

----------------------------------------------------------

Besides pre written ASCII files which can load your memory

slots in an 'instant' from a .mem file, a 'high speed' load is available

for all three hundred slots directly from the keyboard. Try the

following procedure:

1. Do a <^d>, and answer with a <Y> to the prompt. This will clear

and make all 300 slots available for this exercise.

2. Set up on the 20 meter band. Use <F2> and enter 14000000 - or type

<14m> <CR>.

3. Use <F3> and the arrow keys to pick CWW, <CR> - or type <CW><CR>.

4. Hit <^a> or type 'SPEEd slots' <CR>.

5. You are being asked to enter a number.  Enter the frequency

difference in HZ which will be the difference (distance in Hz)

between each slot.  Since we're in the CW portion of 20 meters and

the mode is CWW, let's choose 300 Hz.  Enter 300 - ooops, note as

soon as you hit the digit 3 a 0 popped in to make it a 30. Maybe

it's too smart but it knows the radio can't move 3 Hz so it

supplied a 0. You supply one more 0 to make it 300 and then a

<CR>.

-49-

CAT980                                                 Ver 2.0  Page 50

6. You are now asked how many slots you wish to load. Enter the

number of slots desired up to the amount shown.  Let's ask for 167

which will mean we can examine the CW spectrum between 14 MHz and

14.050 Mhz - and look every 300 cycles!

7. You get a message that it has been done. Do a <CR> and then if

you'd like to look at what you've accomplished do a <^f>.

8. What next? How about a scan? Do a <^v> and accept the default time

with a <CR>. Ah - the scan moves to the left, but there is no

right of 14 Mhz the way we are set up, so it will jump to slot 167

and head back down to slot 1.  If you have an antenna connected

listen carefully. Hear the signals go by?  Pick one out and hit

any key or if you are using AGC sense then hit a key after a

signal is detected.  Now Do an <ESC> to leave the scan and look at

the <^f> display again. You might have to page down to find the

slot you stopped on.  It will be highlighted in reverse video.

This might be a good time to play with the <Backspace> (or <BkSp>)

and <CtrlBackspace> (or ^BkSp> keys. When you're done an <ESC>

will take you out of the scan function.

Standard Memory (and not so standard) Operations

------------------------------------------------

<Del>   'LOck'          Kind of keep in mind how things ran from the

'UNLock'        front panel before you heard of CAT980.  You

have probably reached over and twisted the main

tuning dial only to grunt in disgust because the

radio was in Memory mode.  You have to engage

the SHIFT/LDB button before you can move the

frequency.

Well the same grunt may be needed but from the keyboard

you can lock or unlock the memory function with the <Del> key,

the same result you get with the Shift/LDB button on the FT-980

panel during non CAT operation.  As with the radio this toggle

can only occur when the radio is in the memory mode. (Type 'MEM

<CR>.)

<Ins>   'WRITE'         When you are in the VFO mode you can store the

present operating parameters in the selected

memory channel with the <Ins> key.

One thing the radio won't do but CAT980 will, is to

bring any of the 16 memory channels 'alive' as the VFO

frequency.  Hitting <ShiftF9> will so translate the selected

memory channel into the VFO mode so the operator then has

control of mode, etc. (See above.)

-50-

CAT980                                                 Ver 2.0  Page 51

CHAPTER SIX - THE COMPLETE CAT980 COMMAND KEY SET

We've discussed many of the commands in some detail. Here they

are again starting with the direct frequency commands. And YES, there

are some new ones in this chapter we didn't 'touch' on the earlier

pages.

The commands in this chapter are those commands that require a

key (or two keys) to be struck. If there is a plain text equivalent

command it is shown.  However there are many plain text commands that do

not have a keystroke equivalent.  It is recommended that you study

Appendix H which gives a complete listing of all plain text commands.

This feature is one which is reserved for registered users.

Hit Key        OR TYPE   Notation  CAT980/FT-980 Action

-------------  --------  --------  --------------------

Right Arrow    'DN 10'   <RtArw>   Down 10 hz

'DN xxxx'           Any xxxx divisible by 10

Left Arrow     'UP 10'   <LtArw>   Up 10 hz

'UP xxxx'           Any xxxx divisible by 10

Up Arrow       'UP 100'  <UpArw>   Up 100 hz

Dn Arrow       'DN 100'  <DnArw>   Dn 100 hz

PageUp         'UP 1K'   <PgUp>    Up 1 Khz

PageDown       'DN 1K'   <PgDn>    Dn 1 Khz

Ctrl Lft Arrow 'UP 5K'   <^LtArw>  Up 5 KHz

Ctrl Rgt Arrow 'DN 5K'   <^RtArw>  Dn 5 KHz

Plus           'UP yK'   <+>       Up y KHz (y=10hz to 500KHz)

Minus          'DN yK'   <->       Dn y KHz (y=10hz to 500KHz)

Home           'TOP'     <Home>    Top or bottom of present FT-980 band

End            'BOTtom'  <End>     limits. These limits default to the

regular ham band edges, i.e. 7.3/7.0

for 40 meters, or they can be defined

by the operator using the Tab

controls (^l, ^u, ^t).  <Home> and

<End> keys will honor activated tab

limits.

Ctrl j         'MID band' <^j>     All ham bands and the GEN band has a

mid point frequency. This will get

you there.

Ctrl PageUp    'BANd UP' <^PgUp>   This is the normal FT-980 band change

command. Hit <^PgUp> to advance to

next upper band.

-51-

CAT980                                                 Ver 2.0  Page 52

Ctrl PageDn    'BANd DN' <^PgDn>   Drops to the next lower band.

Hit Key        OR TYPE   Notation  CAT980/FT-980 Action

-------------  --------  --------  --------------------

BackSpace      none      <BkSp>    If 2 to 300 slots are loaded, <BkSp>

will restore the next lower frequency

slot from the last one chosen. BUT if

no slots are loaded then the last

menu selection will be restored.

Ctrl BackSpace none      <^BkSp>   The same as <BkSp> except the next

higher slot is chosen unless the

slots are empty then the next menu

selection is brought up.

Ctrl Home      'EQUal'   <^Home>   Sets GEN=HAM (or HAM=GEN) VFO

frequency equality. This can be

extremely useful in many situations.

If you are operating on 14.233, HAM

vfo, a <^Home> will set the GEN VFO

to the same frequency.  This will

also work for a HAM=GEN match

provided the GEN  frequency to be

matched lies within a ham band.

Ctrl End       'OFFSet'  <^End>    This is an 'external' command.<^End>

toggles the OFFSET display on the

FT-980 only.  It produces no screen

action, but look at the computer

digital display as you try this

command in split operation.

F1             'HELP'    <F1>      Call CAT980 HELP Display - 4 Pages.

F2             'FREQ'    <F2>      Key in any frequency by the digits.

You can use the <Home><End> keys,

arrow keys and the digit keys to set

up a frequency.  Any time a <CR> is

entered the radio will be deployed to

the displayed frequency - provided it

isn't outside the radio's capability.

F3             'MODE'    <F3>      Mode Options Menu. LSB USB CWW CWN

AMW AMN FSK FM

F4             'HAM'     <F4>      HAM vfo / GEN vfo Toggle

'GEN'

-52-

CAT980                                                 Ver 2.0  Page 53

Hit Key        OR TYPE   Notation  CAT980/FT-980 Action

-------------  --------  --------  --------------------

F5             'VFO'     <F5>      VFO / MEMORY / RxM / RxV Menu

'MEM'

'RXM'

'RXV'

F6             'RIT'     <F6>      RIT Menu

F7             'MEMORy'  <F7>      Radio Memory Display Menu

F8             'FILter'  <F8>      I.F. Filter Display Menu

F9             'SCAN'    <F9>      Scan Menu

F10            'WWV'     <F10>     Time Freq. Toggle (WWV 10 MHz)

Hit Key    OR TYPE   Notation  CAT980/FT-980 Action

---------  --------  --------  --------------------

Shift F1   'LOAD'    <ShftF1>  Load in a file (.mem, .mnu or .log)

Shift F2   'SAVE'    <ShftF2>  Save a Freq/Mode/Filter Set

Shift F3   'RESTore' <ShftF3>  This will toggle the operating

frequency/mode/filter settings between

the present set up and any previously

saved by <ShiftF2>.  If no 'SAVE' has

been entered than the default is the

frequency and mode of the entry set when

CAT980 first gained control, or a major

change such as a Band Up command being

issued.

Shift F4   'SHIft xxx' <ShftF4>  Shifts FT-980 I.F. filter down band

by 100 Hz.

Shift F5   'SHIft -xxx' <ShftF5> Shifts FT-980 I.F. filter up band

by 100 Hz

Shift F6   'WIDth xx'   <ShftF6>  Narrows FT-980 I.F. filter right skirt

to the left, toward a lower frequency

-53-

CAT980                                                 Ver 2.0  Page 54

Shift F7   'WIDth -xx'   <ShftF7> Narrows FT-980 I.F. filter left skirt

to the right, toward a higher

frequency

Shift F8   'CENter'      <ShftF8> Centers I.F. filter width and shift.

Hit Key        OR TYPE   Notation  CAT980/FT-980 Action

-------------  --------  --------  --------------------

Shift F9       none      <ShftF9>  Translates present FT-980 memory

channel into the vfo mode.

Shift F10  'ALTernate'   <ShftF10> Activates secondary time freq.

Alt F2         none      <AltF2>   Toggles screen display between the

previous Ver 1.8/1.9 screen displays

and Ver 2.0 series displays. The way

CAT980 displays the Operating

Frequency is affected the most.

Alt F3         'CAT980'  <AltF3>   Toggles between YAESU's filter

'FT980'             configuration and CAT980's shifted

filter configuration.

Alt F4         'FSK'     <AltF4>   Toggles filter configurations between

'AFSK'              relationships based on the FT-980 FSK

tones or config file supplied AFSK

tones.

Alt F5         AGC ON'   <AltF5>   Toggles the AGC sensing capability of

'AGC OFF'           CAT980 for scanning control.

Alt F6         none      <Alt F6>  Toggles the display of the file box

information.

Alt F7         'CAL'     <AltF7>   Used to quickly set up any AGC level

you wish to use for the scanning

function. CALibration is explained

both on the screen and in Appendix G.

Control R      'PICk'    <^r>      This restores any slot frequency

previously by a <^s> or loaded with a

'high speed' load.  Simply supply the

slot number when prompted.  If the

exact slot number is not known use

<^f> for a display of all loaded

slots.

-54-

CAT980                                                 Ver 2.0  Page 55

Control Q      'STATUS'  <^q>      This activates the FT-980 and CAT980

status display. When you hit a <^q>

there is a slight delay. This is

because CAT980 is sending a request

to the FT-980 to return ALL 148

status bytes. This is to ensure that

the info about to be presented on

your screen is correct.  Note that

the left hand side box is FT-980

status. That which is in effect at

the moment is highlighted in a

different color. The info in the

right box on the screen pertains to

CAT980 itself. This includes stored

info and file names.

Hit Key        OR TYPE   Notation  CAT980/FT-980 Action

-------------  --------  --------  --------------------

Control K      'AUTO'    <^k>      Automatic Band Tabs Toggle: this

'AUTO ON'           command engages the band tabs so that

'AUTO OFF'          they will be set automatically - NOT

TURNED ON, BUT THE FREQUENCIES SET

- to the limits of each band.  The

limits themselves are set by program

defaults or statements imbedded in

.mnu files. If a <^t> is entered then

the tabs will be turned on and

honored for each band change.

Control T      'TABs'    <^t>      Toggles activation of the present TAB

set.

Control L    'SET Lower'  <^l>     Sets lower TAB to present OP FREQ.

Control U    'SET Upper'  <^u>     Sets upper TAB to present OP FREQ.

Control W  'CLEAR Tabs'   <^w>     Reset Band Tabs to Program Defaults.

'RESET Tabs'            On 40 meters, for instance, this

would be 7.0 MHz/7.3MHz. This may

also be accomplished by a statement

inside a .mnu file.

Control O      none      <^o>      This command toggles the continuous

Tx monitor control on and off.  When

on, CAT980 CONTINUALLY interrogates

the radio to see if it has changed

from its last state of receive or

transmit.  This can be handy in VOX

operation when there is no way for

the program to know that the

operator's voice has keyed the radio.

-55-

CAT980                                                 Ver 2.0  Page 56

If you are using a lash up where the

RTS or DTR line is keying the radio

then this feature should be left in

it's default OFF state.

A word of caution about enabling this feature: because of the

steady stream of data between the computer and radio, the other commands

become slower acting.  If a command key is held down to slew a frequency

change, the buffer in the computer may fall behind the radio to the

point that control is lost!  Depending on your operating habit this may

or may not be a problem.  Experiment.  But if you don't need the feature

it's recommended that you leave it disabled. Imagine the CPU demand on

your poor PC! (Like Dr. Frankenstein, I'm sorry I turned this guy on.)

The same horror may be also be enabled by a line in the .cfg

file.  See Appendix A.

Hit Key    OR TYPE       Notation  CAT980/FT-980 Action

---------  --------      --------  --------------------

BackSlash  'VFO'         <\>       This is a Quick Memory/Vfo Mode

'MEM'                   Toggle.

CtrlSlash  'RIT          <^\>      Yields a Quick RIT (Rx) On/Off

toggle.

'RIT ON

'RIT OFF'

Control E  none          <^e>      This allows toggle VFO/MEM/PROMPT

options for Memory-VFO switching

operations.  When you switch to

Memory from VFO control you can have

CAT980 go directly to the FT-980

channel chosen. But you may wish to

automatically install the channel

frequency/mode into the VFO upon

selection.  Or maybe you would like

to be prompted as to installing the

channel into the VFO each time the

Mem control is called. <^e> toggles

through all 3 possibilities and if

you have a permanent choice it may be

initialized by the .cfg file.  See

Appendix A.

Tilde      'TUNE'        <~>       TUNE!! Key the transmitter without

<`>       10,000 steps.  This command does the

following:

1.  Saves your present vfo mode of

operation.

2.  Puts the FT 980 into FSK mode.

3.  Sets the RTS line high.

-56-

CAT980                                                 Ver 2.0  Page 57

4.  Starts a 20 second watch dog

timer.

5.  Restores the original parameters

of operation when the timer gets

to 0.

After the 30 seconds or if you hit <AnyKey> before that time,

the RTS line will go low and the mode returned to whatever it

was initially.  If you haven't interfaced your key line to

the RTS line than you must push the FT-980 MOX button to key

the transmitter or use VOX.  And this might be a good time to

consider implementing the RTS feature, it's really part of

CAT philosophy. See Appendix G.

There are limitations: this feature is not available if you

are in Split Operation (RXV or RXM). Also if you are in

Memory mode then TUNE will only work if the channel you have

selected is in the FSK mode.

Hit Key    OR TYPE       Notation  CAT980/FT-980 Action

---------  --------      --------  --------------------

Right Shift none         <RtShift> In SPLIT operation, here's a quick

way to listen to the TX frequency

before you transmit; called  'The

DXer's Key'. If you are running in

Split operation then you can spot

check the transmitting frequency by

holding down the Right Shift key.

When you do so only the frequency is

changed - the mode remains.  So if

you are transmitting on 7.180 Mhz LSB

and listening for stations on 7.079

MHz LSB you can hold down the Right

Shift key and hear 7.180 LSB - but

only as long as you keep the key

depressed.  As soon as you release it

you are back into split operation.

Since only the frequency is changed

this is an instant way to spot check

before you transmit.

Repeater splits on 10 meters can make use of this key in a

rather unique way to see if the other station is coming into

the area on the input frequency and perhaps suggest that a

direct QSO is possible on simplex.

Control X  'DIR'         <^x>      DIRECTORY:  This brings up the same

'DIR *.MNU'             screen as if a "DIR *.*" was done

'DIR *.MEM'             from DOS which will show all the

'DIR *.LOG'             files in the directory where CAT98O

resides. The 3 main CAT980 type files

-57-

CAT980                                                 Ver 2.0  Page 58

(.mnu, .mem and .log) can be viewed

as specific sets.

TIP:  PUT A COPY OF THE DOS FILE MORE.COM IN THE SAME DIRECTORY FROM

WHICH YOU ARE RUNNING CAT98O.EXE. THIS WILL KEEP INFO FROM SCROLLING OFF

OF YOUR SCREEN IF THE DIRECTORY IS LARGE. (CAT980 will detect and use

the file lf it is present).

Hit Key    OR TYPE       Notation  CAT980/FT-980 Action

---------  --------      --------  --------------------

Control Z 'DOS'         <^z>      Allows you to enter a DOS Shell.

After you do a ^z you will be in the

DOS Shell which may give you the

impression that CAT980 is 'gone' but

it really is still in memory just

waiting for you to return.  In this

state you should be able to execute

DOS commands (DIR's, rename files -

even run other programs.

(DISCLAIMER: MAYBE - in any case do

NOT attempt to run a program which

will use the same port that is

controlling your radio!).  When you

are ready to return to CAT98O make

sure you are at the DOS prompt and

simply type the word EXIT and back it

comes. A DOS Shell should not be

confused with running a resident

program (TSR) though there are some

similarities.

You can also enter the DOS Shell by

choosing <X> after initiating a

program exit sequence with <AltX>.

Hit Key    OR TYPE       Notation  CAT980/FT-980 Action

---------  --------      --------  --------------------

Alt C      'SHOw COLors'

<AltC>    Reset Colors. Since you have control

over changing colors when you load

your menu (.mnu) files, you just

might reach a state where you have a

hodgepodge of colors you can't stand.

Do an <AltC> and a color chart comes

up showing you the colors vs their

letter equivalent. The 'BOX' colors

are important; the statements like

'Box0=W y' are the ones you use when

-58-

CAT980                                                 Ver 2.0  Page 59

you write your own files and have

caused the present colors.

Notice that the command line is prompting you to reset all

colors to their default. If you choose 'Y' then all the

colors will be reset and the new ones displayed.  <AnyKey>

drops the display.

These default colors looked good to this op when I wrote the

program with a CGA monitor.  Now that I have a VGA, - and

allowing Windows 3.1 to do its thing, welllll - maybe its

time for a change. Anyhow you can set the initial colors

yourself if you desire. This display will help you pick these

colors. Please see Appendix E for how to do it.

Hit Key    OR TYPE       Notation  CAT980/FT-980 Action

---------  --------      --------  --------------------

SpaceBar   none         <sp>       Activate User Menu Display.  Just hit

the space bar and up pops the present

active menu.  If all you want to do

is look and no selections are to be

made, simply tap the space bar again

or <ESC> and the menu will drop out.

If you have not loaded any menu from

the .cfg file than you may do so with

<ShftF1> or the 'load fname' command.

EXTERNAL COMMANDS

=================

You may want to control some device from within CAT980,

especially the transmit function.  Both RTS and DTR lines can be

controlled from the keyboard.

Hit Key    OR TYPE       Notation  CAT980/FT-980 Action

---------  --------      --------  --------------------

TAB        'TX'          <TAB>     This command toggles the RTS line

high and low while <TX> insures RTS

is high. It is assumed you will want

to use this pin to put the FT-980 in

the transmit mode.

'RX'                    This insures that RTS will be low for

receive operations.  See Appendix G

for a schematic on how to use this

function.

Shift TAB  none          <ShftTAB> Toggles the DTR line high and low.

CAT980 is unaware of your intentions so it is taking no

chances. If either of the commands above are initiated, CAT980

-59-

CAT980                                                 Ver 2.0  Page 60

interrogates the FT980 for a complete status to see if there has been

any changes.  What it's looking for mainly is to see if the transmitter

has been turned ON.  A byte in the FT-980 status stream contains this

info.  If you use the <TAB> key to raise the RTS line and key the radio,

your command line prompt will change from the normal "COMMAND >" to a

blinking "TX CMD >>" indication.

-60-

CAT980                                                 Ver 2.0  Page 61

CHAPTER SEVEN - GETTING STARTED IN WRITING CAT980 FILES

BEFORE YOU START WRITING ANY FILES....

======================================

Before you attack the Appendices which will guide you to writing

various files, a few words about writing files in general would be

appropriate.

As you know many editors add a lot of control bytes that the

editor itself uses.  Most editors have an ASCII save function to write a

file without including these characters.  If nothing else the EDLIN

(yukk) editor that comes with DOS will create usable files for

CAT980.(Update: DOS 5.0 has a super easy ASCII editor.)  Borland's

SideKick (tm) editor is a good one and Notepad in MS Windows (tm) will

do nicely.

FORMATTING RULES FOR CAT980 FILES

---------------------------------

1. Statements may be upper, lower or mixed case unless

noted.

2. Statements must appear with the correct syntax and be

spelled exactly:

freq=2810000 is valid, frequency=2810000 will be

ignored or worse, 'misunderstood'.

3. Statements must contain NO spaces:

channel=11 is a valid statement; channel= 11 is not

valid because of the space between '=' and '11'.

4. Statements should be on the same line, separated by

spaces, unless noted.

5. All statements after an asterisk * will be considered a

remark and ignored.

6. All frequencies in all files must be seven (7) digits

long:

2810000 will be read as 28,100,000 Hz.

BUT 28100000 which is 8 digits will go off into

never-never land.

Comments In The File - Play It Safe, Use A Leading Asterisk

----------------------------------------------------------

Any text in the file NOT preceded by an asterisk (*) will be

considered a command statement. If a line does start with an asterisk

then it will be ignored and thus can be useful for comments. CAT980 will

examine each line and will attempt to execute anything it does not

consider a comment.  That's why it's good practice to always put an

asterisk at the beginning of a comment line. As soon as CAT980 spots one

-61-

CAT980                                                 Ver 2.0  Page 62

it will move on to the next line and go about it's business. Not just

'good practice' but consider it a 'must do'.

Load CHANNEL = 12 for cw net. (This will be ignored as a

comment. But not a good idea.)

*Load CHANNEL = 12 for cw net. (This will definitely be ignored!)

Load CHANNEL=12 for cw net. (This WILL load channel 12! but Cat980

will still try to make sense out of

the comment 'for cw net.')

Load CHANNEL=12 *for cw net. (OK - but still not the best...)

����������������������������������������������Ŀ

� Load CHANNEL=12                              �

� *for cw net. (Now this is the way to do it!  �

������������������������������������������������

Error Messages For The Files You Write - Maybe

----------------------------------------------

CAT980 tries to catch specific errors if you miswrite a

menu, memory of config file.  When the file in question is called upon

that's when the messages may pop up.  Watch for them!

HINTS ON WRITING FILES

-----------------------

The more comments and blank lines you have in any .mem or user

file you create, the longer it will take for CAT980 to 'read' it. BUT

keep in mind, if the program 'sees' an asterisk it will ignore the rest

of the line following it and thus get on with it.  But a lot of bla bla

bla without an asterisk will not only slow the process down somewhat but

it presents the danger that CAT980 will think it understands some of the

bla bla and try to execute it!  So use the asterisk * !  Many of todays

computers are so fast that 'reading' time may no longer be a problem for

many folks. Besides it's a good idea to use all the comments you can so

you can share your files with others.

The supplied files on your disk contain a number of comments

worth reading.  These files are intended to serve as a tutorial aid as

well as be functional.  So please take the time to look at them and

print 'em out for a little study.

Keep your files simple at first. Just get them to do what you

basically want them to do.  Later you can experiment and add color

changes, turn the 'beep' on and off, etc. It will take time to learn it

all but it's part of the fun. Appendix F has been included as a quick

aid in helping you to write your files.

-62-

CAT980                                                 Ver 2.0  Page 63

OK, now head for the Appendices; they will be your CAT980

'graduate studies'.

And consider sharing your menus and memory files with other CAT980

users.  Put 'em on CompuServe and other bulletin boards.  You may even

send your disk to me and I'll share 'em with others.

----------------- CAT980 REGISTRATION INFORMATION ------------------

Today, hams around the world are using CAT980. As a group they

have provided many of the features found in this version and many of

their ideas are still tacked to my wall for future code writing orgies.

Newsletters have also been going out to registered users (now

and then, no guarantee - hi hi) to keep everyone informed. As these keys

are hit I can say again, "the Fat YL has not sung yet!" - I'm having too

much fun.

Registered users get first crack at future versions. Sometimes

there may be an upgrade fee required, sometimes there may not be. But

reason will prevail in this area.  I will make an honost attempt to

answer any letters and help you solve any problems that may arise.

I must add that it is not clear to this author where CAT980 is

going.  As of this writing the thrust is toward a new animal that will

be a true Windows application (Version 3.0 series).  I welcome your

comments in this regard.

-63-

CAT980                                                 Ver 2.0  Page 64

If you bought this version thru CompuServe or a similar net it is

suggested that you still mail in the following form.  Check the 'Paid

Via' box and note the source of the download.

============================================================

|  SHAREWARE DONATION     SUGGESTED                          |

|  CAT 980 REGISTRATION.  $20.00 Continental USA             |

|                         $25.00 Overseas                    |

|                          _                                 |

|                         |_| Paid Via __________________    |

|                                                            |

| NAME ____________________________________________________  |

|                                                            |

|                                                            |

|                                                            |

| ADDRESS__________________________________________________  |

|                                                            |

|                                                            |

| _________________________________________________________  |

|                                                            |

|                                                            |

| CALL_______________________(if any, or put SWL)            |

|  __                                  __                    |

| |__|   5 1/4" disk                  |__| 3 1/2" disk       |

|                                                            |

|  __            __                    __         __         |

| |__| 360K     |__| 1.2M             |__| 720K  |__| 1.44M  |

|                                                            |

|                                                            |

|  SEND TO:                                                  |

|                                                            |

|  CAT980 / K5YEF                 COMMENTS & SUGGESTIONS     |

|  P.O. BOX 861719                WELCOMED FROM ALL          |

|  PLANO, TX. 75074  U.S.A.                                  |

|                                                            |

============================================================

ACCESS NEW VERSIONS ON COMPUSERVE: GO SWREG

SEARCH ON ID 72770,2653

AND ALWAYS CHECK HamNet FOR ANY FREE GOODIES!

-64-

CAT980                                                 Ver 2.0  Page 65

SOURCE CODE OFFER

=================

The source code for version 1.8 is now available to registered users.

Version 1.8 is 'pure' Turbo Pascal and can be compiled by TP version

4.0. (Perhaps even version 3.0; I don't have that version anymore so I'm

not sure.) (It is intended to eventually release Ver 1.9 source code as

well.)

============================================================

|  SOURCE CODE OFFER      YOU MUST BE A REGISTERED USER      |

|  CAT 980 S/W V1.8       TO ORDER THE SOURCE CODE           |

|                                                            |

|                                                            |

|                                                            |

| NAME ____________________________________________________  |

|                                                            |

|                                                            |

|                                                            |

| ADDRESS__________________________________________________  |

|                                                            |

|                                                            |

| _________________________________________________________  |

|                                                            |

|                                                            |

| CALL_______________________(if any, or put SWL)            |

|                                                            |

|  __                                                        |

| |__|  Ver 1.8 (Turbo 4.0)                                  |

|       $12.00 U.S.                                          |

|       $14.00 Elsewhere                                     |

|  __                                  __                    |

| |__|   5 1/4" disk                  |__| 3 1/2" disk       |

|        360K                              720K              |

|                                                            |

|  SEND TO:                                                  |

|                                                            |

|  CAT980 / K5YEF                 COMMENTS & SUGGESTIONS     |

|  P.O. BOX 861719                WELCOMED FROM ALL          |

|  PLANO, TX. 75074  U.S.A.                                  |

|                                                            |

============================================================

IMPORTANT:

It is the author's intention to release Ver 1.8 source code to the

Public Domain sometime after August 1993 and be available 'free'.

-65-