EASY FAX instruction manual

===========================

GENERAL INFORMATION

-------------------

EASY FAX is a non-professional radio facsimile tool.  Completely

developed  using the Turbo C language this software  is  intended

for free exchange, any use of this software for commercial aim is

forbidden.

Unlike  others software for facsimile reception, EASY  FAX  does

not  require  any type of interface to connect the audio  of  the

receiver  to  the  computer;  in each  case  ,  to  increase  the

sensitivity  of the system, the insertion of a signal-squarer  is

to  prefer.  To receive amplitude modulated  sub-carrier  signals

(such   as  weather-satellite  signals)  a   voltage-to-frequency

converter is required; it must be preceded by a band pass  filter

tuned on the sub-carrier frequency (usually 2400Hz or 1200Hz),the

output  of this one have to be rectified before the  introduction

into  the same converter. More detail on the  hardware  explained

later.

The computer must be IBM 100% compatible with at least  640kbyte

of  memory, 1 hard disk, 1 Very enhanced Graphics  Array  adapter

board  (VGA), 1 RS232 serial adapter. Intel 80286,80386 or  80486

microprocessor-based  computers are prefered, especially  if  the

240  lines per minute reception is request, on the  8088/8086  or

very   slow  80286  computers  the  software  frequency   sampler

implemented  by the EASY FAX program may not allow  reception  of

higher lines per minute fax signals.

Probably EASY FAX can run with MS-DOS 2.0 or later, in each case

an MS-DOS 3.0 or later is to be prefered.

INSTALLATION PROCEDURE

----------------------

To run correctly, EASY FAX has to be copied into a sub-directory

of  your  hard  disk, once this operation is done,  you  have  to

decide  where is the place to put the image files, i.e.  in  what

sub-directory  or disk you want to store it. The mean to  specify

this  sub-directory  is the enviromental  variable  EF_PATH.  The

set  of EF_PATH can be done into your autoexc.bat file  including

the  line :  SET EF_PATH=[disk id:][directory path]; for  example

if you want to place your received images into the  sub-directory

\EASYFAX\IMAGES   of  disk  drive  E:,  you  must  add  in   your

autoexec.bat this line :

SET EF_PATH=E:\EASYFAX\IMAGE

All  disk identifier is valid, in each case the use of a  floppy

disk  such  as A: or B: isn't a good idea, due to the  slow  data

flow  who  they can support. The lenght of the  path  name  can't

exceed  26  characters.  If  you  prefer  instead  of  the  above

operation, you can create a batch-file like this explained below:

SET EF_PATH=E:\EASYFAX\IMAGE

EASYFAX %1

SET EF_PATH=

Running  this  file  (called  for  example  EF.BAT)  instead  of

EASYFAX.EXE,  you  can  release  the  memory  occupied   by   the

enviromental variable, once the EASY FAX session is ended. The %1

parameter is useful to permit the access to the EASY FAX setup.

Remember that the EF_PATH variable specify the default value, if

its setup is missed, the sub-directory where the EASYFAX.EXE file

is placed is assumed.

EASY FAX SETUP UTILITY

----------------------

The  first time you run EASY FAX probably you have to  set  some

parameters  in function of your hardware; some of these  such  as

the  communications port, the interrupt handler,  the  brightness

and the contrast of video display are accessible only by mean  of

the  /S switch added in the command line; i.e. to enter in  setup

mode you have to type the command show below:

EASYFAX /S

After this command is done  EASY FAX show you the current  setup

and wait for changes. In detail the keys actived in this  section

are the followings:

<F1>  = change the communications port (COM1:,COM2:,COM3

and COM4 are availables) the  interrupt  handler

is set to the conventional value  for  the  port

selected.

<F2>  = change the interrupt handler (IRQ3 or IRQ4).

<Left>/<Right> = move the graphic cursor inside the gray-scale to

select the gray bar to modify.

<Down> = darken  the selected bar.

<Up> = brighten the selected bar.

<Esc> = accept the changes for this session of  EASY FAX

but don't store them on disk.

<Enter> = accept the changes and  store them on  disk  for

next sessions of EASY FAX.

The  brightness  and darkness limits for each gray  bar  is  the

value  of the two bars respectively on the right and on the  left

of the bar selected.

HARDWARE CONNECTIONS

--------------------

Once  decided what's the communications port to connect  to  the

radio,  the  pins of the connector on the rear  of  computer  are

assigned as described in the table below:

SIGNAL | DB25 PIN # | DB9 PIN # | FUNCTION

--------+------------+-----------+----------

GND   |   1 & 7    |     5     |  GROUND

--------+------------+-----------+----------

RTS   |     4      |     7     |   +12V

--------+------------+-----------+----------

DTR   |    20      |     4     |   -12V

--------+------------+-----------+----------

DSR   |     6      |     6     | AF INPUT

--------+------------+-----------+----------

How  you can see in the table, are indicated two columns of  pin

numbers, this is done because of the two standard connector  used

in  RS232  network, so if your computer is equiped with a  9  pin

connector  you  have  to refer to the DB9 column,  else  if  your

computer is equiped with a 25 pin connector you have to refer  to

the  DB25  column. The current sourced by the RTS  and  DTR  pins

don't  have  to  exceed 5mA, so if your  interface  needs  higher

current  you  have to provide an external power supply;  in  this

case  remember  to connect its ground to the GND  pin(s)  of  the

computer.  Many people thinks that connect a radio to a  computer

is dangerous for electrostatic discharge, i don't advise the  use

of  photocoupler  but  if you want to put it  between  the  rigs,

remember  to  unconnect the ground wire of computer by  the  some

wire of the radio (or interface), else the photocoupler  function

is  bypassed  and  void.  How discussed  at  the  begin  of  this

document, the use of a signal-squarer is advisable to improve the

sensitivity   of  the  system;  it  may  be  realized  using   an

operational amplifier used such as a Schmitt trigger with a small

voltage  hysteresis  (less than 100mV);  an  another  inexpensive

signal-squarer  may  be  done simply driving the base  of  a  NPN

bipolar  transistor  with  the AF signal via  a  current  limiter

resistor  (about 10Kohms), the emitter must be connected  to  the

GND pin, the collector instead must be connected to the DSR  pin,

a pull-up resistor(1Kohm) must connect RTS to DSR, this  solution

theorically is wrong, but i had test it on some RS232  interfaces

with good results.

HOW TO USE EASY FAX

-------------------

This software was written for fun, it's not a professional  tool

for  radio  fax  reception, but it can be a good  aid  for  radio

relayers  or listeners who want to approach to the world of  slow

scan  image  transmission (meteo chart, newspaper  and  satellite

photos).

To  start  EASY FAX in DOS environment execute  EASYFAX.EXE  (or

EF.bat if any). Once the EASY FAX logo is appeared on your  video

screen, you have to press one key to continue; after you can  see

the  "EASY  FAX main menu" appear on the top-left corner  of  the

video  screen. This main menu can be cleared or recalled via  the

<Esc>  key  (it's usually located at the top-left corner  of  the

keyboard).

The  "EASY FAX main menu" shows the commands availables, on  the

left  of the equal sign is represented the key to be pressed  and

on  right is indicated a short form description of  the  function

associated to the key.

Near  the bottom of the "EASY FAX main menu" is present  a  gray

box,  in this box are normally displayed : the  current  selected

lines per minute scan rate, the page wrapping option status,  the

name  of the image loaded into the video buffer and the  name  of

the setup file in use.

MAIN MENU FUNCTIONS

-------------------

<R>  (image  Reception)

This command lets you enter into the "EASY FAX rx menu". If  you

have received another image yet, EASY FAX prompt you to answer if

you  want really continue (in that case you lost last  image)  or

return to main menu. Once you are in the rx menu the reception is

enabled and the signal received (if any) begin to build the image

starting  on the top-left corner of the video screen.  In  bottom

left  corner  the  rx menu remember you  what  are  the  function

enabled.  This  menu  is slowly cleared by the  image  that  take

shape, this to let it the longer time possible on the screen. The

sub-functions enabled are :

<Home> (syncro start) pressing this key the computer enter  into

a  waiting  state  until the standard  syncronization  signal  is

received.

<End> (abort syncro start) clear the above function.

<Del>  (clear  screen) erase the entire video screen  and  video

buffer, after it redraw the "EASY FAX rx menu".

<F1> (LPM select) pressing this function key change the line per

minute rate, the values for this selection are: 60,90,120,180 and

240 (Note  that higher lines per minute rate may be  unaccessible

on slow machines).

<W>  (turn  page wrapping on|off) this key, if pressed, let  you

switch  on or off the capability of the program to return at  the

top-left  corner of video after the bottom-right of the  same  is

reached(if  this function is disabled the program exit  from  the

"EASY  FAX rx menu" when the image reach the bottom of the  video

screen).

<F4> (update file ########.DAT) using this function key, you can

update the configuration file indicated(if any).

<Left> (\\ image adjust) this key lets you straighten the  image

during reception if it's top-left to bottom right inclined.

<Right>  (// image adjust) the same as the above  mentioned  but

for top-right to bottom-left image inclination.

<+>  (\ fine image adjust) it's the same of <Left> but for  fine

tuning.

<-> (/ fine image adjust) it's the same of <Right> but for  fine

tuning.

<Esc> (return to main menu) this key,if pressed exit from the rx

menu and go back to the "EASY FAX main menu".

<T>  (Tuning utility)

This  key  clear the image from the video screen (not  from  the

video  buffer) and put you into the tuning tool; it's a  spectrum

analizer like display. On the top-left of the video screen  three

box itemize what's the edge of the gray scale you can move  using

the  <Left> and <Right> keys; to select the box you have  to  use

the  <Up>  and  <Down> keys, the selection is  indicated  by  the

enhancement of the line who composing the same box. How indicated

on the screen, the frequency markers are intended for  indicative

use  only, i.e. may be that the frequency indicated differs  from

the frequency really connected to the serial port, this is due to

the slowness of the computer, in each case the relation from  the

frequency indicated and the gray bar is correct. A special marker

is  also indicated on the graph, this marker shows you the  lower

limit that guarantee the horizontal dot resolution, the  position

of  this marker changes on the 240 LPM scan rate. Tune the  black

and  white bars respectively near the lower and higher  edges  of

the shapes drawn by the signal received.

<O>  (set Origin)

Using  this key, the "EASY FAX help" window appears  instead  of

the  main menu; this window tells what you have to do. To  remove

the  help window you have to press any of <Left>,<Right>,<Up>  or

<Down>  key or, if present, move your mouse on the desk; now  you

can  move the pointer (the small cross appeared on the centre  of

the  screen)  using the above keys or the mouse; if you  use  the

keys  remember that the <Scroll Lock> key sets the speed  of  the

pointer  movements, in detail if the Scroll Lock light is  turned

on the speed is one pixel at a time otherwise is eight pixels  at

a time. The pointer indicates the pixel who will become the  top-

left corner of the video screen, to do that press the <Enter> key

otherwise, to abort this function press the <Esc> key. For  mouse

users the left mouse button is the same of <Enter> and the  right

mouse button is the same of <Esc>.

<H>  (Horizontal image mirror)

This function is useful in many facsimile signals who build  the

images  from right to left instead of the standard left to  right

image  transmission. Once you pressed this key the image  on  the

screen is re-painted.

<V>  (Vertical image mirror)

This  function re-paint the image up side down, it's useful  for

polar weather satellite images that represent the earth depending

on their path (if North to South or vice versa).

<N>  (Negate (reverse) color)

This key, if pressed reverse the gray shades i.e. black  becomes

white  and  vice versa etc. This is useful, for example,  if  you

mistake the sideband during the reception of an SSB image.

<Z>  (Zoom selected zone)

Using  this key, like in the "set origin", the "EASY  FAX  help"

appears  instead of the main menu; the operation are the same  as

such  in the set origin function; the difference is the  pointer,

who is a 320 by 240 pixels box; once you pressed the <Enter>  key

or  clicked the left mouse button, the part of image  inside  the

box is magnified by 2, otherwise you can press <Esc> or click the

right  mouse  button to shut down this function.  Note  that  the

magnified   image  maintain  the  vertical  resolution  and   the

horizontal  resolution  is re-computed via the mean  of  adiacent

pixels.

<S>  (Save image)

Once you pressed this key, EASY FAX ask you the name to give  at

the  image to be saved, if you answer with <Esc> the function  is

aborted  and nothing is done. The name may be 8 characters  long,

and must be at least 1 character, all alphanumeric keys are valid

characters,  the extension is by default ".FAX" and the  path  is

set  by the EF_PATH enviromental variable or the later  discussed

<C> command. To accept the name and save the image press <Enter>.

<L>  (Load image/List image)

This key puts the program into the "EASY FAX image loader"; here

you  can load one of the listed images files simply pressing  the

respective  function  key  associated to,  if  the  sub-directory

contains other images you have to press the <F10> key to  display

them.  The small gray box at the bottom of the image loader  show

the  current  sub-directory  and the name of  the  current  image

loaded into the video buffer. If you want to shut down the  image

loader without loading any image, press the <Esc> key.

<P>  (create 1280x960pixels Pxc image)

Once you pressed this key, the program ask you the name to  give

at the ZSoft .PCX file to be created. The keys and procedures are

the same as in the "Save image" function except for the extension

(naturally  is  .PCX  !!). This function is  useful  to  transfer

images to high level drawing programs such as PaintBrush VI  plus

by ZSoft, these programs can clear the images from spots produced

by  weak  signals  received and can print the  image  using  very

powerful functions.

<C>  (Change current path for images)

This function lets you free to change the current path name i.e.

the  place where EASY FAX stores and load the images.  The  maxim

length is 26 characters, all alphanumeric keys are valid here  in

addition the <\> and <:> characters are available; the <Esc>  key

shut  down the function and leave the path unchanged,  to  accept

the  new  path name press the <Enter> key; root  directories  are

invalid  and  floppy  disk identifier such as A: or  B:  are  not

advisable.  An  invalid path name produces an error  message  and

assumes the sub-directory where the EASY FAX program is placed as

the current path.

<U>  (Up load setup)

After you recall this function the program enter into the  "EASY

FAX setup loader"; it's like the image loader but it show you the

list  of  setups availables. Into the gray box at the  bottom  is

indicated the current setup (if any).

<W> (Write new setup)

This  function write the current settings into a .DAT file.  The

procedure  is the same as the (Save image) function but the  path

is always the sub-directory where the EASY FAX program stays.

<Q> (Quit program)

Once  you  pressed this key, the program ask if you  are  really

sure  and  if you answer <Y> the program exits otherwise  if  you

press <N> it return to main menu.

THANKS

------

I wish to be thankful for the help gived to me by the people who

had  tested the program in its first release. Special  thanks  to

TRAVAGLINO Mario (IK1GKJ) for the consultancy on Turbo C and 8086

assembly  languges  and  BOZZOLA  Angelo  (IK1MSL)  for   program

testing and delivery.

SOMETHINGS ABOUT THE PROGRAM

----------------------------

I began the writing of EASY FAX on March 1991, first it was just

a  bet  with  myself, to be able to  receive  a  facsimile  image

without  any interface; after, many days later, i  have  realized

that  the  program  was  efficient and i have  decided  to  do  a

complete tool for facsimile reception. This was my first  program

written  in C language and it was completed part-time  along  the

rest of 1991, so it is not surely optimized. I hope you will have

fun with it. I wish you all the best in facsimile reception,  and

if  you  are  a radio relayer i hope to met you on  the  air,  my

callsign is IK1IZA.

Best regards

PORZIO Massimo

TRADEMARK ACKNOWLEDGEMENTS

--------------------------

Turbo C is a trademark of Borland International,Inc.

IBM  and VGA are trademarks of International  Business  Machines

Corp.

Intel,8086,8088,80286,80386   and   80486  are   trademarks   of

Intel Corp.

MS-DOS is a trademark of Microsoft Corp.

PaintBrush VI plus and ZSoft are trademarks of Zsoft Corp.