PC/FlexNet Documentation       FLEXNET.EXE       (C)1995 Gunter Jost, DK7WJ

Translation: Tom Sailer, HB9JNX

Date: 16.06.95

FlexNet-Kernel

This is the central FlexNet module. It must always be loaded first. It may be

used in conjunction with FLEXDIGI, if digipeater services are required.

All FlexNet channel drivers are supported. There may be up to 15 channels,

which should suffice for all purposes.

-   Hardware and software requirements

PC/XT or preferably AT with at least 256 kB RAM

PC/FlexNet needs about 60 kB RAM plus the RAM needed for channel

drivers and utilities.

-   Operating system

MS-DOS version 3.1 or later, preferably 5.0 or 6.2, where parts of

the package may be loaded into the UMBs

-   Interfaces

as required (see available channel drivers)

-   A PC/XT should basically suffice. The achievable data rate depends largely

on the speed and throughput of the channel drivers.

-   Tests with MS-DOS 6.0 showed problems, and there is no experience with

DR-DOS. The author suggests the use of MS-DOS 5.0 or 6.2. This allows

the modules to be loaded into UMBs if there is sufficent memory.

EMM386 or foreground programs with virtual memory management may in some

cases lead to loss of data on the channels, but this depends largely on

the computer and the channel drivers that are used.

-   To load the modules (with MS-DOS 5.0 or later), use "LOADHIGH" or "LH",

it doesn't hurt; if there is not enough memory in the UMBs, the modules

are loaded into the conventional memory. Even in this case memory can be

saved, since the environment block does not leave a gap in the memory.

"MEM /D" shows the memory usage.

-   PC/FlexNet supports loadable L1 drivers (or channel drivers). They are

installed in memory simply by calling them. Therefore it is very easy to

support any interface. A "driver kit" is available for interested

developers.

-   Channel numbers are allocated in ascending order of the driver

installation. A driver may support any number of channels. FlexNet however

supports at most 15 channels.

-   The whole installation is usually done with a batch file, because most

channel drivers need command line parameters. Error conditions are

reported back to COMMAND.COM, and the batch file should terminate the

installation.

-   FLEXNET.EXE must always be loaded first. Then the channel drivers

should be called in the desired order. Then the modules must be activated

by calling the utility "FLEX". After activation, channel driver

installation is refused. Now, channel parameters must be set up using

the "FSET" utility. The mode parameter is driver dependent and may be

found in the driver documentation or the online help, which is displayed

with "<driver name> /?".

Applications (terminal programs and so on) or the hostmode emulator TFEMU

may now be loaded.

-   The size of the used data memory may be specified as an optional command

line parameter to FLEXNET.EXE. A reasonable minimum is 15 kB per active

channel. If there are many QSOs (for example BBS), there should be at least

30-40 kB, more does not hurt.

-   Channel drivers are included in the package if available. Every channel

driver comes with a .DOC file with the same name that explains its

installation. Every driver also displays a short help text if called with

/? that explains its parameters.

-   FLEXNET.EXE, the channel drivers and other extensions may be deinstalled

by calling "FLEX.EXE /U". DOS however requires that there were no other

TSRs loaded after FLEXNET, otherwise the machine may crash.

-   New in version 3.3e: An integrated multi channel digipeater. The call

sign is entered with "FSET" (see below). The output channel is built

from the digi SSID by adding the desired channel number. Now it is

possible to build cross channel gateways without the need for the full

featured FLEXDIGI node.

An example batch file to start the PC/FlexNet package follows (comments at the

end of the lines after the * must be omitted!)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> cut here <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

LH FLEXNET 20               * Kernel with 20kB data memory

if errorlevel 1 goto end    * optional: if already installed, give up!

LH SER12 2                  * channel 0: BayCom serial modem at COM2

if errorlevel 1 goto error  * optional: if an error occured, give up!

LH KISS 1                   * channel 1: KISS mode to connect a computer at COM1

if errorlevel 1 goto error  * optional: if an error occured, give up!

FLEX                        * activate the kernel and the channel drivers

FSET mode 0 1200c           * mode channel 0: 1200 baud half duplex, soft DCD

FSET txd  0 15              * txdelay channel 0 150ms

FSET mode 1 38400cd         * mode channel 1: 38400 baud full duplex, CRC mode

FSET digi AC5FC             * optional: set digipeater callsign

LH TFEMU                    * optional: start hostmode emulator

goto end

:error

FLEX /u                     * if an error occured, unload the rest

:end

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> cut here <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Optionally, futher resident applications may be started, see the accompanying

documentation.

At last the forground application may be started if needed. The kernel

stays active even if the application is terminated.

The whole package may be unloaded with "FLEX /U".

-   If FLEXNET is used as a non-digipeater (i.e. without loading FLEXDIGI),

only one channel is used normally, so the port needs not be specified.

If there are multiple channels, the following should be noticed:

The default channel for outgoing connects is channel 0. If another

channel should be used, the channel number must be specified as first

digipeater, for example:

"CONNECT DB0ODW 1"

or

"CONNECT DK7WJ via 1 DB0ODW"

The "via" is optinal, as usual.

The applications determines if the port number of incoming connects

is used or displayed.

- Digipeating: If the given digi callsign is AC5FC-3, a frame "via AC5FC-3"

will be output on channel 0, "via AC5FC-4" will come out on channel

1 and so on.

Useful utilities:

FSET     Sets channel parameters (TX Delays and baud rates etc)

SHOW     Traces channels and displays the channel states (PTT/DCD)

STAT     Displays channel statistics

CALIB    Switches the calibration mode on and off

Every utility displays a command line parameter help if called with "/?".

-------------------------------------------------------------------------------

If you have questions or problems, please contact first the one from whom

you got the package. Often, the local packet node sysop may help, especially

if it is a FlexNet node.

Please contact the author only if the above failed, by sending a mail to

DK7WJ @DB0ZDF. The answer may take a few days. If questions are asked that are

answered sufficiently in the documentation or that can be found by

experimenting, the mail may fall into the bit bucket.

This software package is distributed freely. If you like the package and you

want to support the work of the FlexNet group, a donation is welcome.

The money will be used for further hardware and software development for

RMNC and FlexNet.

����������������������������ͻ

� Sonderkonto FlexNet-Gruppe �

�    Sparkasse Darmstadt     �

�    Kto.Nr. 111 050 090     �

�    BLZ      508 501 50     �

����������������������������ͼ

Note: This is a translation of the german legal terms. It is for informational

purposes only, they have, however, no legal significance. The german terms

are the only applicable terms.

The following legal terms govern the use of PC/FlexNet and its modules.

Different modules from other authors may use different terms, which should

be read in the documentation of the modules!

-   RMNC/FlexNet, PC/FlexNet and the accompanying utilities and documentations

are a product of Gunter Jost, DK7WJ. Exceptions are marked. All rights

remain at the author. The user has the right to use the software under

the following conditions:

-   The software may only be used noncommercially for amateur radio. Every

other use, especially in other radio services, needs in every case the

written consent of the author.

-   The legal regulations concerning amateur radio are met.

-   Commercial copying and selling without the written consent of the

author is prohibited. Also, the software must not be bundled with

commercially sold components (for example modems) without the

written consent of the author. In every case the customer must be

informed in writing that he does not acquire further rights concerning

the software.

-   The software must not be patched or changed. Changing parameters

however is allowed.

-   Copyright notices must not be removed or changed from the software

modules and documentation.

-   The software must not, neither in part nor wholly, published over

PBBS systems or publicly accessible mailboxes.

-   The software may be distributed freely. One must, however, always

distribute the complete package, without any change.

File dates and times must not be changed, so that different versions

can easily be distinguished.

Own extensions have to be clearly marked, at least with the callsign.

(Note: This paragraph does not apply to the network node version!)

-   The authors or distributors of this software assume no responsibility

or liability for any harm that results from using this software.

The use of this software implies accepting the above legal terms.

------------------------------------------------------------------------------